AART Slot Car Braking Module

Design Document

Target MCU: STM32G051K6U6 (QFN-32) | Firmware: Rev 5 | libopencm3

Revision 1.0  |  AART — Adrian & Richard’s Technologies

# 1. Overview

This document describes the hardware design and firmware architecture for the AART slot car braking module. The module sits in-line between a conventional hand controller and the track, providing variable braking, freewheel, and antibrake functions under the control of a single centre-detent potentiometer.

## 1.1 Purpose

Conventional slot car hand controllers apply braking using a fixed or potentiometer-adjusted resistor shorted across the motor terminals. This approach has three fundamental limitations that this module is designed to overcome:

- A conventional wirewound or carbon potentiometer cannot introduce the very small, precise changes in resistance needed for fine brake adjustment. The mechanical and electrical resolution of a pot is far too coarse for competitive tuning, particularly with brushless motors where even small differences in braking characteristic are significant. Electronic control of a MOSFET in its linear region provides resolution several orders of magnitude finer than any mechanical pot.

- Group 12 brushed motors with high Kv ratings generate very large back-EMF pulses. Even a modest resistance brake can produce violent, destabilising deceleration. These motors need not only finely adjustable braking but also the ability to apply a small forward voltage (positive anti-brake) that counteracts the braking torque, smoothing the deceleration profile so the car settles into corners rather than snapping.

- Brushless motor installations use an eCom (electronic commutator) that has its own onboard microcontroller. Conventional brake circuits work by shorting all power from the track — which also cuts power to the eCom MCU, causing it to reset and lose motor commutation lock. This is not a braking problem as such; it is a side effect of how conventional brake circuits work. The module solves this by monitoring the track voltage and preventing it from falling below the eCom’s minimum operating voltage, so the eCom MCU stays alive throughout the brake event.

The module addresses all three problems in a single external unit that is compatible with any conventional hand controller and requires no modification to the controller or the car.

## 1.2 System position

The module connects via 4mm banana plugs to the three standard track sockets and exports matching sockets for the hand controller to plug into. It is fully transparent during normal running and takes over braking when the controller shorts the track.

| Banana colour | Signal | Description |
| --- | --- | --- |
| WHITE | +12–16V always-live rail | From track PSU. Can deliver 40–90A. Powers the module and Q1 injection. |
| BLACK | Track positive (+ve to car) | Controlled by hand controller. Monitored by module to detect brake condition. |
| RED | Common negative | Left braid return. Module and track ground reference. |

The module presents identical WHITE/BLACK/RED sockets on its output side for the hand controller to plug into. During normal running, BLACK passes through unmodified. When the hand controller shorts BLACK→RED (brake condition), the module detects this by voltage sense on the BLACK terminal and applies its own controlled braking.

The WHITE rail is always live at 12–16VDC regardless of throttle position. The hand controller varies how much of this rail reaches the BLACK output. The module draws its MCU supply from WHITE via the LDO. WHITE is also the source for Q1 (anti-brake injection and keep-alive floor).

## 1.3 Circuit topology

The diagram below shows the principal signal paths through the module. All three rails (WHITE, BLACK, RED) run left to right. The hand controller connects on the left; the track and motor are on the right.

[Figure 1: Circuit topology diagram — see AART_Brake_Topology.svg delivered alongside this document]

Key points from the diagram:

- D1 (LTC4412 pass gate) sits in the BLACK controller input path — not the WHITE rail. During normal running it is fully transparent (near-zero voltage drop). During brake takeover it opens, isolating the controller’s short from the track.

- Q1 (P-ch, high-side) connects WHITE to the BLACK_to_track node. It provides positive anti-brake voltage injection in modes A/B and the keep-alive voltage floor in mode C. Its gate is driven by a linear mode gate driver (op-amp buffer) from DAC2.

- Q2 (N-ch, low-side) connects BLACK_to_track to RED/GND. It provides the variable brake resistance from approximately 8Ω to a near dead short. Its gate is driven by a linear mode gate driver from DAC1.

- Both FETs operate in their linear (triode) region — no PWM switching. The DAC sets a DC gate voltage; the FET presents a continuously variable drain-source resistance. No 20kHz switching, no radiated EMI, no interference with brushless eCom electronics.

- The MCU senses WHITE dV/dt (for Latvian brake detection), V_BLACK_to_track (for brake entry/exit hysteresis), and the pot and mode switches. It generates DAC1 and DAC2 voltages accordingly.

- The R10/R11 voltage divider sensing BLACK_to_track is on the motor side of D1 — this is essential so the MCU can detect the voltage collapse that occurs when D1 opens and the module takes over braking.

# 2. Operating Modes

The module has three operating modes selected by a 3-position toggle switch. Within each mode the brake is triggered by voltage sense on the BLACK terminal rather than a mechanical wiper strip contact, making the module compatible with any hand controller.

## 2.1 Brake detection — voltage sense with hysteresis

The BLACK terminal voltage is continuously monitored via an ADC input. A hysteresis band prevents chatter at low throttle settings near zero:

- V_BLACK < 500mV → enter brake state (controller has shorted BLACK→RED)

- V_BLACK > 1500mV → exit brake state (controller delivering power)

- 500mV ≤ V_BLACK ≤ 1500mV → hold current state

This approach works with any hand controller regardless of whether it has an accessible wiper strip contact, and correctly handles partial braking (controllers that do not short fully to zero).

## 2.2 Mode A — Brushed motor with antibrake

| V_BLACK | Pot position | Q1 (anti-brake) | Q2 (brake) | Behaviour |
| --- | --- | --- | --- | --- |
| > 1500mV | Any | OFF | OFF | Pass-through — module transparent |
| < 500mV | Full CCW | PWM — ~8Ω | OFF | Softest brake |
| < 500mV | CCW → centre | PWM ramping | OFF | Brake resistance decreasing |
| < 500mV | Centre ±deadband | Fully ON | OFF | Dead short — max braking |
| < 500mV | Centre → CW | OFF | PWM 0→2V | Antibrake injection ramping up |
| < 500mV | Full CW | OFF | PWM — ~2V | Maximum antibrake |

## 2.3 Mode B — Brushed motor, brake only

| V_BLACK | Pot position | Q1 | Q2 (brake) | Behaviour |
| --- | --- | --- | --- | --- |
| > 1500mV | Any | OFF | Always OFF | Pass-through |
| < 500mV | Full CCW | PWM — ~8Ω | Always OFF | Softest brake |
| < 500mV | Full CW | Fully ON | Always OFF | Dead short — max braking |

## 2.4 Mode C — Brushless motor, eCom keep-alive

In mode C, Q1 holds the BLACK_to_track voltage at or above the saved keep-alive floor level. This prevents the eCom MCU from seeing a brownout during braking. The eCom is powered by the hand controller during normal running — Q1 only activates when V_BLACK would otherwise drop below the floor setpoint.

| V_BLACK vs keep-alive | Wiper state | Q1 (keep-alive) | Q2 (brake) | Behaviour |
| --- | --- | --- | --- | --- |
| V_BLACK > KA setpoint | Not braking | OFF | OFF (suppressed) | Controller powering eCom — Q1 not needed |
| V_BLACK ≤ KA setpoint | Not braking | OFF | ON at saved CCR | Keep-alive active — eCom powered by module |
| V_BLACK < 500mV | Braking | PWM per pot | ON at saved CCR | Brake + keep-alive in parallel |
| Capture button held | Either | OFF | Pot-controlled | User dials keep-alive voltage |

During a hard brake (Q2 dead short), V_BLACK collapses toward 0V. Q1 is active trying to maintain the keep-alive floor but the low impedance of Q2’s dead short will pull the floor down. D4 is essential in mode C — it blocks reverse current through Q1 when Q2 holds the track near 0V, protecting Q1’s body diode from the repetitive peak current.

# 3. Hardware Design

## 3.1 Component list

| Ref | Component | Value / Part | Notes |
| --- | --- | --- | --- |
| U1 | MCU | STM32G051K6U6 | QFN-32, 3.3V, 64MHz |
| U2 | LDO regulator | AMS1117-3.3 | WHITE rail → 3.3V, SOT-223 |
| GD1 | Gate driver | UCC27524 | Keep-alive / anti-brake FET channel (Q1), SOT-23-5 |
| GD2 | Gate driver | UCC27524 | Brake FET channel (Q2), SOT-23-5 |
| Q1 | Anti-brake / keep-alive FET | IRF9Z34N P-ch | High-side switch, 18A, D2PAK |
| Q2 | Brake FET | IRLB3034 N-ch | 195A, 1.4mΩ Rds(on), D2PAK |
| D1 | TVS diode | P6KE18CA (bidirectional) | Rail transient clamp, 18V, SMB. CA suffix essential — bidirectional variant clamps both polarity transients from inductive switching. |
| D2 | Schottky | SS310 | Q2 catch diode, 3A/100V SMB. 100V reverse voltage gives margin over SS34’s 40V on a 16V rail with inductive spikes. |
| D3 | Schottky | SS310 | Q1 catch diode, 3A/100V SMB. Same upgrade as D2. |
| D4 | Schottky | SS34 | Mode C series output diode — ESSENTIAL |
| D5 | LED red | 0805 | Status indicator PB6 |
| F1 | Polyfuse | 500mA–1A, 1812 | WHITE rail supply protection for MCU/LDO only. Must NOT be in the BLACK (track) path — a polyfuse cannot handle 40A motor startup and will trip from thermal lag. The BLACK path carries full motor current; PSU overcurrent protection covers that rail. |
| R1 | 10kΩ | 0402 | NRST pull-up |
| R2 | 10kΩ | 0402 | GD2 input pull-up (safe default brake) |
| R3 | 10Ω | 0402 | Q2 gate resistor |
| R4 | 10Ω | 0402 | Q1 gate resistor |
| R5 | 10kΩ | 0402 | Pot top-end to 3.3V |
| R6 | 10kΩ | 0402 | Capture button pull-up PA5 |
| R7 | 1kΩ | 0402 | Capture button series to GND |
| R8 | 15kΩ | 0402 | WHITE sense top divider (PA0) |
| R9 | 4k7Ω | 0402 | WHITE sense bottom divider (PA0) |
| R10 | 22kΩ | 0402 | BLACK sense top divider (PA1) |
| R11 | 4k7Ω | 0402 | BLACK sense bottom divider (PA1) |
| R12 | 330Ω | 0402 | Status LED current limit |
| C1–C4 | Decoupling | 0805/0402 | 100nF + 10μF per LDO rail |
| C5–C7 | Decoupling | 0402 | 100nF VDD, 100nF VDDA, 4.7μF VDDA |
| J1–J3 | 4mm banana sockets | Track side | WHITE / BLACK / RED to track |
| J4–J6 | 4mm banana sockets | Controller side | WHITE / BLACK / RED from hand controller |
| J7 | 3-pin 2.54mm | Pot connector | 3.3V / wiper / GND |
| J8 | 3-pin 2.54mm | Mode toggle | Bit0 / Bit1 / GND |
| J9 | 2-pin 2.54mm | Capture button | Momentary pushbutton |

## 3.2 Pin assignments — STM32G051K6U6 QFN-32

| Pin | Signal | Type | Function |
| --- | --- | --- | --- |
| PA0 | WHITE sense | ADC_IN0 analog | WHITE rail ÷ R8/R9 divider. V_PA0 = V_WHITE × 0.2386. Safe to 16V. |
| PA1 | BLACK sense | ADC_IN1 analog | BLACK track terminal ÷ R10/R11 divider. V_PA1 = V_BLACK × 0.1760. Brake detection. |
| PA2 | Toggle bit 0 | GPIO input, pull-up | LOW = mode B or invalid. Decoded with PA4. |
| PA3 | Pot wiper | ADC_IN3 analog | 10k pot, 0–3.3V. |
| PA4 | Toggle bit 1 | GPIO input, pull-up | LOW = mode C or invalid. Decoded with PA2. |
| PA5 | Capture button | ADC_IN5 analog | 10k pull-up; button → GND via 1k. ~4095 unpressed, ~372 pressed. |
| PA6 | Brake PWM | TIM3_CH1 AF1 | Q2 gate via GD2. INVERTED polarity. CCR=0 → dead short. |
| PA7 | Keep-alive PWM | TIM3_CH2 AF1 | Q1 gate via GD1. Normal polarity. CCR=0 → Q1 off. |
| PB6 | Status LED | GPIO output | Blinks 3× on keep-alive save to flash. |

## 3.3 Voltage dividers

Both sense inputs use the same bottom resistor (4k7Ω) for component commonality. The top resistor differs to accommodate the different maximum voltages:

| Signal | R_top | R_bot | Ratio | V_pin at 16V rail | Safe? |
| --- | --- | --- | --- | --- | --- |
| WHITE (PA0) | 15kΩ | 4k7Ω | 0.2386 | 3.82V — marginally over | Add 12k top for 2.96V (see note) |
| WHITE (PA0) revised | 18kΩ | 4k7Ω | 0.2073 | 3.32V | ✓ safe to 3.3V rail |
| BLACK (PA1) | 22kΩ | 4k7Ω | 0.1760 | 2.82V | ✓ safe to 3.3V rail |

If the track PSU can deliver up to 16V, use R8=18kΩ (not 15kΩ) for the WHITE sense divider. Update WHITE_DIV_NUM/DEN in brake_module.h to 47/234 accordingly. The original 15k value was sized for 12.4V supplies; at 16V it produces 3.82V on PA0 which exceeds the 3.3V ADC reference.

## 3.5 Power and thermal analysis

The following analysis confirms component ratings against the worst-case electrical demands of the application. Track PSU: 12.4V nominal, 16V maximum. Motor startup: 40A for a few milliseconds. Braking back-EMF: 20A+ momentarily. PSU cannot absorb reverse current.

| Component | Worst-case demand | Rating | Verdict | Action taken |
| --- | --- | --- | --- | --- |
| Q2 IRLB3034 (brake FET) | 20A braking current continuous during brake P = I² × Rds = 400 × 0.0014 = 0.56W ΔT = 35°C above ambient (no heatsink) | 195A, 40V, 175°C Tj Rds(on) 1.4mΩ Rth_ja 62°C/W | ✓ Comfortable | No change. Motor startup current (40A) does not flow through Q2 — Q2 is OFF during throttle. Copper pour under D2PAK pad recommended. |
| Q1 IRF9Z34N (keep-alive FET) | Mode C keep-alive: 5–50mA (eCom load) — negligible Simultaneous Q2+Q1: up to 46A instantaneous through Q1 body diode if D4 absent | 18A, 55V, 175°C Tj Rds(on) 350mΩ @ Vgs=−3.3V | ✓ OK (with D4) | D4 mandatory in mode C. Without D4, body diode of Q1 carries 46A repetitively at 20kHz — not rated for this. D4 blocks this current path. |
| D1 TVS (P6KE18CA) | Motor back-EMF adds to rail: 16V + 12V peak = 28V transient Energy: <<600W × 1ms pulse capacity | 600W peak (10/1000µs), Vc=25.2V @ 1A | ✓ OK | Changed to bidirectional CA variant. Unidirectional P6KE18A only clamps one polarity; inductive switching generates both. |
| D2/D3 SS310 (catch diodes) | Q2 turn-off spike: L × dI/dt = 100µH × 20A/1µs = 2000V unclamped D2 clamps at Vf + Vdc ≈16.4V, energy = 20mJ | 3A avg, 100V, same SMB footprint as SS34 | ✓ Upgraded | Changed from SS34 (40V) to SS310 (100V). D2 is essential — without it Q2’s 40V Vds is instantly exceeded on turn-off. 40V reverse rating left no margin on 16V rail. |
| D4 MBRS360 (mode C series diode) | Repetitive ~46A pulses at 20kHz during mode C hard braking (Q1 ON-time into near-shorted track via Q2) | 3A avg, 60V, 150A peak (single pulse) Same SMB footprint | ✓ Upgraded | Changed from SS34 (80A peak, marginal under repetitive 20kHz stress) to MBRS360 (60V, same SMB footprint). SS54 (5A, 150A peak) is equally suitable. D4 position is the most electrically stressed diode in the design. |
| F1 polyfuse (WHITE rail) | MCU + LDO + gate drivers ≈ 35mA total on WHITE rail No motor current on WHITE rail during braking or throttle | Resettable fuse, now 500mA–1A | ✓ Corrected | Downrated from 25A to 500mA–1A. F1 protects the MCU/LDO supply on WHITE only. It must NOT be in the BLACK (track) path which carries 40A motor startup and 20A braking current — a polyfuse would trip. |
| U2 AMS1117-3.3 (LDO) | Load: ~35mA P = (16 − 3.3) × 0.035 = 0.44W at 16V rail ΔT = 0.44 × 62 = 27°C (SOT-223, no heatsink) | 1A, 125°C Tj, Rth_ja 62°C/W | ✓ OK | Tj = 52°C at 25°C ambient, 97°C at 70°C ambient. Both within 125°C limit. Solder SOT-223 tab to copper pour for better thermal performance. |

Motor startup current of 40A does not flow through any module component during normal operation. Q2 and Q1 are both OFF when the hand controller is delivering power (V_BLACK > 1500mV). The module is fully passive during throttle. Only during braking does current flow through Q2, and only then does the back-EMF need to be managed.

## 3.6 User interface — control devices

The module has four physical control devices. All are on the module enclosure itself; no modification to the hand controller is required.

| Control | Type | Connector | Function |
| --- | --- | --- | --- |
| Brake pot | 10kΩ linear rotary, centre detent | PA3 (ADC_IN3) | Sets brake resistance and anti-brake voltage. In modes A and B: CCW = softest brake (~8Ω effective), CW = hardest brake (dead short). In mode A: CW past centre detent = positive anti-brake, 0–2V. Centre detent in mode A marks the dead-short / anti-brake boundary. In mode C: full range maps to brake resistance only. |
| Mode toggle | 3-position switch | PB0 (bit 0), PB1 (bit 1) | Selects the operating mode. Position A: brushed motor with positive anti-brake. Position B: brushed motor, brake only. Position C: brushless motor with eCom keep-alive floor. The toggle is read at each 1ms tick with debounce. |
| Capture button | Momentary pushbutton, normally open | PB3 (GPIO) | Dual function depending on mode. In mode C: hold and rotate pot to set keep-alive floor voltage; release to save. In modes A and B: hold and rotate pot to set brake ramp-in time (CCW = 0ms instant, CW = 200ms maximum); release to save. LED blinks 3× on save. Values are written to flash and survive power cycles. |
| BOOT0 button (J12) | Momentary pushbutton, normally open | PB8/BOOT0 | ROM bootloader entry. Hold BOOT0 and press NRST to enter the STM32 factory UART bootloader. Used for field firmware updates without an ST-Link programmer. Not part of normal operation. |

Pot rotation direction and feel:

- The centre detent is the most important tactile reference for mode A. It marks the transition from braking (CCW side) to positive anti-brake (CW side). A pot without a centre detent in mode A makes it impossible to reliably find the transition point by feel.

- In modes B and C the detent still provides a useful mid-range reference but is not architecturally significant.

- Pot resolution: at 300° total travel, 12-bit ADC, and a 600mV DAC control window for Q2: each degree of rotation corresponds to approximately 10mV change in Q2 gate voltage, or roughly 0.05–0.2Ω change in effective brake resistance depending on position. See section 15 for the full resolution analysis.

Capture procedure summary:

- Mode C keep-alive: connect to track PSU. Hold capture button. Rotate pot CW slowly until the eCom powers up and motor begins to twitch. Back off slightly. Release button. LED blinks 3×.

- Modes A/B ramp time: hold capture button. Rotate pot to desired ramp time (CCW = fast, CW = slow). Release button. LED blinks 3×. Typical starting points: wing cars 80–150ms, pan cars 0–30ms.

## 3.4 Safe default behaviour

On power loss or MCU fault, the brake FET (Q2) must default to ON (hard brake). This is achieved by:

- A 10kΩ pull-up resistor (R2) on the GD2 input. When the MCU output goes high-impedance (reset/power-off), R2 holds GD2 input HIGH → driver output HIGH → Q2 gate HIGH → FET on → hard brake.

- TIM3_CH1 is configured with INVERTED polarity so CCR=0 produces a HIGH output during normal operation, consistent with the pull-up logic.

- brake_force_safe() writes directly to TIM3 CCR registers and is called from hard_fault_handler and nmi_handler.

# 4. Firmware Architecture

## 4.1 File structure

| File | Description |
| --- | --- |
| Core/Inc/brake_module.h | Constants, pin map, types, public API |
| Core/Src/brake_module.c | Control logic, ADC/DMA, PWM, flash storage |
| Core/Src/main.c | Top-level: SysTick ISR, fault handlers, main loop |

## 4.2 Timing and scheduling

The module uses a polling architecture. brake_tick() is called from the main loop as fast as possible and uses a millisecond counter (incremented by SysTick ISR) for internal pacing. ADC conversions run via DMA so the CPU never blocks waiting for a result.

## 4.3 ADC configuration

ADC1 is configured in scan mode with four channels in sequence:

- Rank 1 — PA0 (CH0): WHITE rail voltage sense

- Rank 2 — PA1 (CH1): BLACK track voltage sense

- Rank 3 — PA3 (CH3): pot wiper

- Rank 4 — PA5 (CH5): capture button ADC

DMA transfers all four results to a circular buffer in a single transfer. The libopencm3 ISR dma1_channel1_isr() sets a flag; the main tick reads the buffer on the next 1ms iteration. Sampling time is 39.5 ADC cycles — adequate for the 10kΩ pot and voltage divider source impedances.

## 4.4 DAC configuration

TIM3 runs at 20kHz (above the audio band, good for logic-level FETs):

SYSCLK = 64 MHz, PSC = 0, ARR = 3199

Frequency = 64,000,000 / (0+1) / (3199+1) = 20,000 Hz

CH1 polarity: INVERTED (OCPolarity_LOW)

CCR = 0    -> output HIGH -> brake FET ON  (hard brake)

CCR = 3199 -> output LOW  -> brake FET OFF (no brake)

CH2 polarity: NORMAL (P-channel Q1 — gate LOW = FET on)

CCR = 0   -> output LOW  -> Q1 OFF (no injection)

CCR = 400 -> PWM 400/3200 = 12.5% duty -> ~2V at 16V rail

Note: effective voltage scales with WHITE rail voltage

Recalculate KA_CCR_MAX if PSU voltage changes: CCR = V_target/V_rail x 3200

## 4.5 Key constants

| Constant | Default | Meaning |
| --- | --- | --- |
| ADC_CENTRE | 2048 | Midpoint of 12-bit ADC range |
| ADC_DEADBAND | 150 | ±3.7% of range — freewheel zone around centre |
| BRAKE_DUTY_HARD | 0 | CCR for full hard short (FET fully on) |
| BRAKE_DUTY_SOFT | 2720 | CCR for ~8Ω effective resistance at pot minimum |
| ANTI_DUTY_MAX | 516 | CCR for ~2V antibrake injection at 12.4V rail |
| WIPER_DEBOUNCE_MS | 10 | Wiper strip debounce time in milliseconds |
| ADC_RAIL_DIVIDER | 0.2386 | Voltage divider ratio R3/(R2+R3) for rail sense |
| RAIL_UNDERVOLTAGE_MV | 5000 | Below this rail voltage — force safe brake state |

## 4.6 Control flow

Each 1ms tick executes the following sequence:

- Update ADC readings from DMA buffer if conversion complete

- Check rail voltage — force safe brake if below RAIL_UNDERVOLTAGE_MV

- Read debounced wiper strip state (PA1)

- If wiper open: set MODE_PASSTHROUGH, both FETs off, return

- Read pot ADC value and compare against deadband thresholds

- Below deadband: compute brake duty, update TIM3_CH1, set antibrake to 0

- Above deadband: compute antibrake duty, update TIM3_CH2, set brake to full off

- Within deadband: both FETs off — MODE_FREEWHEEL

# 6. Bench Testing

## 6.1 Brake detection verification

The brake trigger is V_BLACK based. To simulate brake conditions on the bench, apply a variable voltage source to the BLACK input terminal and verify PA6/PA7 respond correctly:

| V_BLACK applied | Expected PA6 (brake) | Expected PA7 (Q1) | State |
| --- | --- | --- | --- |
| 0V (short to RED) | HIGH or PWM per mode/pot | Per mode | Brake active |
| 0.3V | HIGH or PWM | Per mode | Brake active (below 500mV threshold) |
| 0.5–1.5V | Holds previous state | Holds previous state | Hysteresis zone — no change |
| 2V | LOW (if was in passthrough) | Per mode C / OFF for A/B | Brake released |
| 8V (throttle mid) | LOW | OFF (A/B) or OFF if suppressed (C) | Pass-through |
| 16V (full throttle) | LOW | OFF | Pass-through, Q1 suppressed in mode C |

## 6.2 Power budget

At 20A motor current with IRLB3034 (Rds(on) = 1.4mΩ typical):

P_Q2 = I² × Rds(on) = 400 × 0.0014 = 0.56W (hard short, full current)

P_Q1 = I_keepalive² × Rds(on)  — negligible (eCom load is milliamps)

P_LDO = (V_WHITE − 3.3V) × I_MCU = (16 − 3.3) × 0.02 = 0.25W typ

Well within D2PAK ratings for Q2. The LDO dissipation increases at higher rail voltages — at 16V consider a small heatsink on U2 or a pre-regulator resistor to drop some voltage before the LDO.

# 7. Open Items and Known Limitations

| Item | Status | Notes |
| --- | --- | --- |
| Q2 gate drive at 3.3V | Verify on bench | IRF9Z34N Vgs(th)=-2V to -4V. Op-amp buffer must swing low enough to fully enhance Q2. Use rail-to-rail op-amp with output reaching within 50mV of GND. |
| D_PG LTC4412 CTRL pin logic | Verify polarity | Confirm LTC4412 CTRL pin logic: active-low or active-high depending on variant. MCU GPIO must correctly open the pass gate for brake takeover. |
| BLACK sense divider position | PCB layout | R10/R11 (22k/4k7) must be on MOTOR side of D_PG (LTC4412 output), not controller side. Critical for correct brake detection. |
| DAC_BRAKE_SOFT calibration | Bench verification required | Default DAC value 1860 (~1.5V) gives approximately 8Ω. Verify on bench with actual motor. Adjust via SET BRAKE_SOFT <dac_val> over UART. |
| Ramp-in reset on mode change | Low priority | If mode toggle changed mid-ramp, ramp state not cleared. Add mode_prev tracking if needed. |
| Latvian brake — corrected primary use case | Documentation only | Primary use case is car coming out of slot (braid loss), not track power-cut. Trigger logic (WHITE dV/dt) is correct. Consider BLACK dV/dt as alternative trigger for the primary use case in a future revision. |

# 7A. Production Architecture — Design Decisions and Analysis

## 7A.1 Overview

This section documents the key architectural decisions made during the design process, the alternatives considered, and the rationale for each choice.

| Block | Prototype approach | Production change | Reason |
| --- | --- | --- | --- |
| MOSFET pass gate on BLACK | Not present — direct connection | P-ch MOSFET + NPN level-shift (prototype) or ideal diode IC e.g. LTC4412 (production) | Module must be able to pull BLACK to near zero to sense brake condition. Series diode drops 0.4–0.7V at full throttle and dissipates heat at 40A. |
| Linear output stage | PWM brake FET (TIM3_CH1, 20kHz) | FET in linear (triode) region, DC gate voltage from DAC + op-amp buffer | Brushless eComs are sensitive to PWM on braid. CE/EMC regulations restrict high-frequency switching in this application. |
| Keep-alive floor | Q1 injects small voltage | Active clamp prevents BLACK dropping below eCom minimum. Does not supply. | The eCom is powered by the controller. The module only needs to floor the voltage, not supply it. |
| Hold-up capacitor | None — MCU loses power with rail | 220–330µF on 3.3V rail, ~100ms hold-up | Preserves braking continuity after PSU cut, like a conventional brake resistor. Only practical with low-power MCU. |

## 7A.2 MOSFET pass gate — gate drive options

The pass gate requires a circuit that can turn the P-channel MOSFET on and off under MCU control, with the gate voltage referenced to V_BLACK_IN (0–16V) while the MCU GPIO is referenced to GND at 3.3V. Three options were evaluated:

| Option | Circuit | Quiescent current | Body diode handling | Verdict |
| --- | --- | --- | --- | --- |
| A — NPN level shift | NPN transistor with collector to P-ch gate. R1 pulls gate to V_BLACK_IN (FET off). MCU GPIO drives NPN base via R2 — NPN on pulls gate to GND (FET on). | ~50µA | Needs external SS310 Schottky in anti-parallel | Prototype choice. Two transistors, three resistors. Simple and bench-verifiable. |
| B — Bootstrap gate driver IC | High-side gate driver IC with bootstrap capacitor | ~1–5mA | Needs external Schottky | Avoid. Too much quiescent current for capacitor hold-up. |
| C — Ideal diode IC | e.g. LTC4412, MAX40203. Auto-manages gate drive relative to rail. CTRL pin gives MCU override to force pass gate open for brake takeover. | ~20–50µA | Fully managed by IC | Production choice. Single IC, near-zero quiescent, body diode handled. |

The BLACK sense ADC input (PA1, R10/R11 divider) must be placed on the MOTOR side of the pass gate. Only then does the voltage collapse when the pass gate opens, providing the brake detection trigger. In the prototype the sense point is on the controller side — adequate for threshold validation but must move for production.

## 7A.3 Linear mode FET control

In the prototype design Q2 is switched at 20kHz (PWM). In the production design Q2 is biased in its linear (triode) region: gate voltage is set to a DC level between threshold and full enhancement, giving continuously variable Rds(on) without switching noise.

| Vgs (IRLB3034) | Approx Rds(on) | Brake effect |
| --- | --- | --- |
| < 1.5V | Off | No brake |
| ~1.6V | ~8Ω | Soft brake (pot CCW) |
| ~2.0V | ~0.5Ω | Medium brake |
| ~2.5V | ~5mΩ | Near dead short (pot CW) |

The useful control range is ~600mV of gate voltage. Gate voltage generation:

- Prototype: TIM3 PWM + heavy RC filter (10kΩ + 10µF, τ=100ms). Ripple at gate must be <1mVpp. Works with STM32G031.

- Production: MCU DAC + rail-to-rail op-amp buffer. True DC, no ripple, fast response. Requires STM32G051K6U6 (has DAC, same QFN-32 package as G031).

## 7A.4 Hold-up capacitor sizing

Capacitor hold-up requires low quiescent current. The dominant consumer in the current design is the UCC27524 gate driver pair (~10mA). In the production linear-control design these are replaced by a single op-amp buffer (~50µA). Formula: C = I × t / ΔV.

| Parameter | Current design (G031 + PWM) | Production (G051 + linear) |
| --- | --- | --- |
| MCU active current | ~8mA @ 64MHz | ~2mA @ 16MHz |
| Gate driver / buffer | ~10mA (UCC27524 ×2) | ~50µA (op-amp) |
| LDO quiescent | ~5mA (AMS1117) | ~0.5mA (low-Iq e.g. MCP1700) |
| Total | ~23mA | ~2.6mA |
| Cap for 100ms, ΔV=1V | ~2300µF (impractical) | ~260µF → use 330µF standard value |
| Stored energy | 12.5mJ | 1.8mJ |
| Regulatory concern? | Marginal | No — 1.8mJ powers MCU only, cannot drive motor |

A 330µF capacitor at 3.3V stores 1.8mJ. A conventional brake resistor stores zero energy yet produces the same braking effect after power-off. The hold-up capacitor powers the microcontroller control circuit, not the motor. BSCRA and ISRA energy storage regulations address capacitors that can drive a motor after trigger release — this does not apply.

## 7A.5 Production MCU: STM32G051K6U6

The STM32G051K6U6 is the current MCU:

- QFN-32 — pin-compatible with the original STM32G031K6U6 footprint. No PCB change needed.

- 12-bit DAC — required for linear gate voltage control of Q1 and Q2.

- 32KB flash, 18KB RAM, 64MHz Cortex-M0+ — adequate for Rev 8 firmware.

- Available at JLCPCB. Not a special-order part.

- All firmware except DAC initialisation is binary-compatible with G031.

## 7A.6 Power-down, brownout and hold-up capacitor

## 7A.6.1 The problem: braking must survive PSU power-off

A conventional slot car brake resistor is a passive device. When the track PSU is switched off, the resistor continues to brake the car because it requires no power of its own. An active electronic brake module loses its MCU power when the PSU cuts, and without the MCU the DAC outputs collapse and the brake FET behaviour depends entirely on the hardware safe-default pull-up.

The pull-up circuit provides a basic hardware safe default (Q2 enters its braking region without MCU power), but the quality and controllability of that braking depends on the DAC being driven. The goal of the hold-up capacitor is to keep the MCU alive for a short period — 100ms is sufficient — after the PSU cuts, allowing the firmware to maintain the full programmed brake characteristic rather than just the coarse hardware default.

100ms is comfortably longer than the time a car takes to decelerate from full speed to rest at any realistic braking force. It is also orders of magnitude longer than the hold-up time needed for the hardware pull-up to take effect, so the two mechanisms together provide continuous, well-controlled braking through the entire power-off transient.

## 7A.6.2 STM32G051 power supply supervisor: POR, PDR and BOR

The STM32G051K6U6 has an integrated power supply supervisor consisting of three coupled circuits:

| Circuit | Function | Threshold | Behaviour on VDD falling |
| --- | --- | --- | --- |
| POR (Power-On Reset) | Holds MCU in reset during power-up until VDD reaches minimum operating voltage | VPOR ≈ 1.7V (rising, typical) | Not relevant on falling edge |
| PDR (Power-Down Reset) | Generates reset when VDD falls below minimum operating level | VPDR ≈ 1.6V (falling, typical) | MCU reset when VDD < VPDR. Registers lost. DAC outputs collapse. |
| BOR (Brownout Reset) | Programmable additional reset threshold, configured via option bytes. Four levels available. | VBOR1–VBOR4: selectable from ~1.7V to ~2.8V | MCU reset when VDD < VBORx falling threshold. Earlier warning than PDR alone. |

The POR/PDR is always active. The BOR is an option-byte-configured addition that can fire at a higher voltage, giving earlier warning. For this application the PDR threshold (~1.6V) defines when the MCU actually stops executing. The firmware can run correctly down to the minimum operating voltage of 1.7V; below that behaviour is not guaranteed.

BOR is not available in Shutdown mode. For this application the MCU never enters Shutdown mode — it runs continuously in normal Run mode or Low-power Run mode. BOR remains active throughout.

## 7A.6.3 Why the STM32G051 has suitable power consumption

The hold-up capacitor size is set by the total module current draw. The lower the current, the smaller the capacitor needed for a given hold-up time. The G051 is suitable for this application for the following reasons:

| Component | Current (prototype, 64MHz) | Current (production target, 4MHz LP Run) | Notes |
| --- | --- | --- | --- |
| STM32G051 MCU | ~6mA @ 64MHz | ~370µA @ 4MHz | G0 family: ~93µA/MHz in LP Run mode. At 4MHz: 93 × 4 = 372µA. |
| Gate drivers / op-amp buffers | ~10mA (UCC27524 ×2) | ~50µA (rail-to-rail op-amp) | Dominant consumer in prototype. Eliminated in production linear-mode design. |
| LDO (AMS1117 → MCP1700) | ~5mA quiescent | ~2µA quiescent | AMS1117 quiescent is 5mA typical. MCP1700 is 2µA. Significant saving. |
| ADC, DAC, GPIO | ~500µA | ~100µA | ADC scanning 4 channels @ 1ms interval. |
| Total | ~21.5mA | ~522µA | Production total well under 1mA. |

The 4MHz LP Run mode figure deserves explanation. The G051 supports a Low-power Run (LP Run) mode when SYSCLK is reduced below 2MHz, but at 4MHz with the main voltage regulator the current is still dramatically lower than at 64MHz. For the brake control application, 4MHz is more than adequate: the firmware runs a simple 1ms control loop with integer arithmetic, ADC reads, and DAC writes. There is no signal processing or protocol handling that requires high clock rates.

## 7A.6.4 Hold-up capacitor sizing — worked calculation

The hold-up capacitor sits on the 3.3V rail between the LDO output and the MCU VDD pins. When the WHITE rail falls (PSU off), the LDO ceases to regulate and the capacitor becomes the sole power source for the MCU.

The capacitor must supply the load current for the hold-up duration before VDD falls below the PDR threshold (~1.6V). The usable voltage swing is from the regulated 3.3V down to 1.7V (minimum operating voltage), giving a ΔV of 1.6V.

Hold-up formula:  C = I × t / ΔV

Where:

I  = total module current during hold-up  = 522µA = 0.000522 A

t  = required hold-up time               = 100ms  = 0.1 s

ΔV = usable voltage swing                 = 3.3V − 1.7V = 1.6V

C = 0.000522 × 0.1 / 1.6

C = 0.0000326 F

C = 32.6µF

Use standard value: 100µF (3× margin)

Verification at 100µF:

t = C × ΔV / I = 100×10⁻⁶ × 1.6 / 0.000522 = 306ms

More than 3× the required 100ms.

A 3× margin is intentional for three reasons. First, capacitor values have ±20% tolerance (X5R/X7R ceramics) and can be as low as −40% of rated value at full DC bias voltage. A 100µF 6.3V capacitor at 3.3V bias may measure 60–80µF in circuit. Second, capacitor capacitance degrades with age. Third, the load current estimate is approximate and some peripheral current was not accounted for.

## 7A.6.5 Component selection: 100µF 6.3V SMD electrolytic

| Parameter | Value | Rationale |
| --- | --- | --- |
| Capacitance | 100µF | Provides >300ms hold-up at 522µA, 3× margin over 100ms target |
| Voltage rating | 6.3V | Minimum acceptable on a 3.3V rail. 10V for increased reliability and reduced derating. |
| Package | SMD electrolytic (6.3mm diameter typical) or 1210 polymer | Through-hole electrolytics are larger and not suitable for a compact SMD board. Polymer SMD has lower ESR. |
| Stored energy | E = ½CV² = ½ × 100×10⁻⁶ × 3.3² = 0.54mJ | Powers the MCU only, not the motor. Well below any regulatory threshold for stored energy devices. A conventional brake resistor stores zero energy yet has the same braking effect. |

The hold-up capacitor must be placed on the LDO output side of any Schottky protection diode if one is used. If the LDO output is directly connected to VDD without a series diode, the capacitor is placed directly at the MCU VDD/VDDA pins. ST recommends 100nF + 4.7µF decoupling at the VDD pin per the G0 hardware design guide; the 100µF hold-up cap is in addition to these, not a replacement.

## 7A.6.6 Behaviour on power-down

The sequence of events when the track PSU is switched off:

| Time | Event | MCU state | Brake output |
| --- | --- | --- | --- |
| t = 0 | Track PSU switched off. WHITE rail begins to fall. | Running normally | Programmed brake level maintained by DAC |
| t ≈ 0–10ms | WHITE rail falls below LDO dropout voltage. LDO ceases to regulate. C_HU begins to supply VDD. | Running from capacitor. Firmware continues executing 1ms tick. | DAC continues driving Q2 at programmed level |
| t ≈ 0–306ms | VDD falls linearly from 3.3V toward 1.7V as C_HU discharges. | Running. ADC results become less accurate below 2V but brake DAC continues. | Programmed brake maintained |
| t ≈ 306ms | VDD falls below VPOR/PDR threshold (~1.7V). MCU resets. DAC output collapses. | Reset. All registers lost. | DAC collapses. Pull-up circuit takes over. |
| t > 306ms | Pull-up circuit holds Q2 gate at safe-default voltage. | In reset / no power | Hardware safe-default brake — light to medium braking force |

The transition at t ≈ 306ms from firmware-controlled braking to hardware-pull-up braking is seamless in practice: the car has already decelerated to rest well within the first 100ms. The remaining hold-up time is margin. The hardware pull-up ensures that even if the capacitor is smaller than expected, the car is never left coasting freely.

# 7B. Key Design Decisions

This section records the significant architectural choices made during development, the alternatives considered, and the rationale for each decision. This is intended to prevent the same questions being relitigated in future revisions.

| Decision | Choice made | Alternatives considered | Rationale |
| --- | --- | --- | --- |
| Brake trigger detection | Voltage sense on BLACK terminal with hysteresis (500mV enter, 1500mV exit) | Mechanical wiper strip contact from hand controller; optoisolator across controller output | Voltage sense works with any hand controller regardless of mechanical design. Hysteresis prevents chatter at low throttle. Wiper strip requires physical access to the controller internals and would not detect partial-short braking. Optoisolator adds complexity and a component. |
| MCU package | STM32G051K6U6 QFN-32 | STM32G041J6M6 SOIC-8 (original design) | QFN-32 provides all required pins: PA0/PA1 ADC sense, PA4/PA5 DAC outputs for gate control, PB0/PB1 toggle, PB3 capture button. The G051 also provides the 12-bit DAC required for linear mode operation. QFN-32 is solderable with hot air. |
| Firmware library | libopencm3 (no ST HAL, no CubeMX) | ST HAL with CubeMX code generation | Eliminates proprietary toolchain dependency. Entire build chain is open source and reproducible in Docker. Resulting binary is ~4.6KB vs ~20-40KB typical HAL. No ST account or licence required. CubeMX generates fragile boilerplate that is hard to version-control. |
| Q2 brake FET polarity | INVERTED (CCR=0 → FET on = dead short) | Normal polarity (CCR=ARR → FET on) | With inverted polarity, the safe failure mode (MCU unpowered, gate floating) defaults to brake ON via the pull-up resistor on GD2 input. With normal polarity, loss of MCU results in no braking — the car runs freely into a corner. Safety requirement drives this choice. |
| Q1 FET type | P-channel IRF9Z34N (high-side) | N-channel with bootstrap gate driver (high-side); N-channel low-side with reversed injection | P-channel requires no bootstrap circuit. Gate driven directly by UCC27524 from 3.3V rail. N-channel high-side requires gate voltage above drain (above rail) — needs charge pump or bootstrap, adding components and complexity. Low-side injection would require motor re-wiring. |
| Keep-alive suppression | Q1 off when V_BLACK > floor voltage derived from saved_ka_dac and white_mv | Q1 always on (rely on D4 Schottky); fixed voltage threshold | Always-on Q1 fights the hand controller output when V_BLACK is above the keep-alive setpoint. This causes unnecessary current through both Q1 and the Schottky. Dynamic threshold using actual rail voltage scales correctly across different PSU voltages. Fixed threshold would be wrong at non-nominal rail voltages. |
| Brake ramp shape | Exponential (geometric decay: ccr += error × alpha/256 per ms) | Linear ramp; S-curve (two-phase coast then ramp) | Exponential gives fast initial onset (prevents car running wide) followed by a gentler tail (avoids snap-stop). Matches the feel of commercial active-coast controllers. Linear ramp has abrupt onset at entry and at completion. S-curve was considered but adds code complexity with marginal feel benefit. |
| Capture button implementation | GPIO on PB3 (active-low, internal pull-up) | ADC pin (original design, before PA4/PA5 were reassigned to DAC outputs) | ADC approach uses the same scan that reads the pot and rail — no extra interrupt or polling path. ADC threshold is adjustable in firmware without hardware change. Shares one of the available ADC channels (PA5/CH5). GPIO would waste a pin; long-press adds UX complexity. |
| Flash storage | Four 64-bit double-words at page 31: {magic,ka_dac,ramp_ms}, brake thresholds, {brake_soft_dac,latvian}, {release_ms_a,b,c} | Multiple 32-bit writes; EEPROM emulation library | G0 flash requires 64-bit aligned writes. Page 31 reserved by linker so firmware cannot overwrite it. Four double-words store all tuneable parameters. EEPROM emulation would add code size for marginal benefit. |
| Anti-brake voltage ceiling | 2V (DAC_KA_MAX=2480, ~2V at 12–16V rail) | 3V; adjustable via second capture | 2V is at the top of the practical range described in commercial controllers (0.2–2V typical). 3V risks spinning the motor rather than providing gentle anti-brake. A hardcoded ceiling with field capture for the specific value needed gives safety and flexibility. |

# 8. Firmware Build — libopencm3 and Docker Toolchain

## 8.1 Library choice

The firmware was written against libopencm3 rather than the ST HAL. This eliminates the CubeMX dependency, keeps the entire toolchain open source, and produces a significantly smaller binary. The compiled debug image is 4,616 bytes of text — approximately 14% of the available 32K flash.

libopencm3 is included as a git submodule and pre-built inside the Docker image for the STM32G0 family only, keeping the image lean. No ST account, no proprietary downloads, no generated code blobs are required to build the project from scratch.

## 8.2 G0-specific differences from other STM32 families

Several libopencm3 API differences apply specifically to the STM32G0 and are worth documenting here for anyone porting from F-series examples:

- DMAMUX — The G0 DMA controller routes requests through a DMAMUX peripheral. Unlike F0/F1 where the DMA channel-to-peripheral mapping is fixed in hardware, the G0 requires an explicit write to DMAMUX1_C0CR to connect ADC1 to DMA1 Channel 1. In the firmware this is done with a direct MMIO32 write to address 0x40020800 (DMAMUX1 base, channel 0 register).

- ADC scan mode — adc_enable_scan_mode() does not exist for G0. Scan is implicit when the sequence length is greater than one. Set the sequence with adc_set_regular_sequence() and the hardware scans automatically.

- ADC resolution symbol — ADC_RESOLUTION_12BIT (F-series) becomes ADC_CFGR1_RES_12_BIT on G0.

- ADC sampling time symbol — ADC_SMPR_SMP_39DOT5CYC becomes ADC_SMPR_SMPx_039DOT5CYC on G0.

- ADC DMA continuous — adc_set_dma_continue() becomes adc_enable_dma_circular_mode() on G0.

- Timer reset — timer_reset() is not available for G0 timers. Use rcc_periph_reset_pulse(RST_TIM3) instead.

- Flash wait states — The G0 rcc_clock_scale struct takes a plain integer (2) for flash_waitstates, not the FLASH_ACR_LATENCY_2WS macro.

- Linker script — libopencm3 dropped all per-device .ld files in November 2020 in favour of a script generator. The project provides its own stm32g041j6.ld containing the MEMORY block (32K flash at 0x08000000, 8K RAM at 0x20000000) and an INCLUDE cortex-m-generic.ld directive.

## 8.3 Docker toolchain

The build environment is fully containerised and reproducible. The Docker image is multi-stage:

| Stage | Contents | Build command |
| --- | --- | --- |
| base | Debian Bookworm slim + apt packages including OpenOCD 0.12.0 | (base of all stages) |
| toolchain | ARM GCC 13.3.rel1 (arm64 native) + libopencm3 pre-built for G0 | docker build --target toolchain -t aart-brake:toolchain . |
| cubeprog | toolchain + STM32CubeProgrammer (optional, needs ST installer) | docker build --target cubeprog -t aart-brake:full . |

OpenOCD is installed from the Debian Bookworm apt repository rather than built from source. The upstream GitHub release for v0.12.0 does not attach a source tarball (only auto-generated archives without a configure script), making source builds unreliable. The Debian package provides the same version without this problem.

## 8.4 Build workflow

All build, flash, and debug operations are available as simple commands:

# Compile (debug)

docker compose run --rm brake build

# Compile (release, optimised)

docker compose run --rm brake build release

# Flash via ST-Link (from host, recommended on macOS)

st-flash write build/aart_brake.bin 0x08000000

# Or via OpenOCD on the host

openocd -f interface/stlink.cfg -f target/stm32g0x.cfg \

-c "program build/aart_brake.elf verify reset exit"

On macOS, Docker Desktop does not reliably pass USB devices to containers. Build inside Docker, then flash from the host using st-flash (brew install stlink) or host OpenOCD (brew install openocd). The build/ directory is bind-mounted so the .elf/.bin/.hex files are available on the host immediately after a container build.

# 9. Pre-Commissioning Checklist

## 9.1 Hardware checks before first power-on

Work through these checks before applying track supply voltage for the first time:

- Confirm the MCU is STM32G051K6U6 (QFN-32). Verify the part number on the chip — the G051 has DAC peripherals required for linear mode control. Confirm PA4 and PA5 are routed to the op-amp buffers for Q2 and Q1 gate control respectively.

- Verify the WHITE sense divider: R8=18kΩ, R9=4k7Ω on PA0. Measure actual fitted values. At 16V rail this gives 3.31V on PA0 which is within the 3.3V ADC reference. If 15kΩ was fitted instead of 18kΩ, replace it — 15k produces 3.82V at 16V, exceeding the ADC input limit.

- Verify the BLACK sense divider: R10=22kΩ, R11=4k7Ω on PA1. Confirm the divider tap is connected to the MOTOR SIDE of D_PG (LTC4412 output), not the controller side. This is critical for correct brake detection.

- Confirm D_PG (LTC4412) is in the BLACK controller input path. Pin assignments: IN+ from controller BLACK, OUT to BLACK_to_track node. Verify CTRL pin polarity from datasheet — confirm MCU GPIO drives it correctly to open the pass gate for brake takeover.

- Confirm D4 (MBRS360 or SS54) is fitted in series with Q1 output in mode C boards. Without D4, Q1 body diode takes the full braking current pulse when Q2 fires. D4 can be omitted only on mode A/B-only boards.

- Confirm catch diodes D2 (across Q2) and D3 (across Q1) are SS310 (100V). The 40V SS34 is not adequate on a 16V rail with inductive motor loads.

- Confirm F1 (polyfuse 500mA–1A) is in the WHITE rail supply path only, upstream of the LDO. F1 must NOT be in the BLACK track path which carries 40A motor startup current.

- Check the op-amp buffers for Q1 and Q2 gate drive: confirm they are rail-to-rail output types and their outputs can swing low enough to fully enhance Q2 (Vgs threshold ~1.5–2V) and pull Q1 gate below its source voltage.

## 9.2 Scope verification sequence — Mode A (brushed + anti-brake)

Set the mode toggle to A. Connect a variable DC supply to the BLACK input. Leave Q2 and Q1 disconnected for initial signal path checks — probe the op-amp buffer outputs driving the FET gates.

| Step | Condition | DAC2 (Q1 KA gate) | DAC1 (Q2 brake gate) | Pass condition |
| --- | --- | --- | --- | --- |
| 1 | Power on, V_BLACK = 8V (throttle) | 3.3V (Q1 off) | 0V (Q2 off) | Module transparent — both FETs off |
| 2 | V_BLACK drops to 0.3V (brake), pot full CCW | 0V | ~1.5V DC (Q2 onset) | Soft brake — DAC at DAC_BRAKE_SOFT, no switching |
| 3 | Brake active, pot swept CW | 0V | Ramps from ~1.5V to ~2.5V DC | Brake resistance decreasing — smooth DC, no PWM |
| 4 | Brake active, pot at centre detent (mode A) | 0V | ~2.5V DC (near dead short) | Maximum braking |
| 5 | Brake active, pot CW past centre | Ramps from 3.3V toward ~2.0V | 0V (Q2 off) | Positive anti-brake region |
| 6 | V_BLACK rises above 1.5V (throttle pickup) | 0V | Ramps linearly to 0V if RELEASE_MS_A > 0 | Trail brake release — state = RELEASE then PASS |
| 7 | Remove MCU power | 3.3V (Q1 off) | ~1.5V+ via pull-up circuit | Safe default — Q2 enters braking region |

## 9.3 Scope verification sequence — Mode B (brushed, brake only)

Set the mode toggle to B:

| Step | Condition | DAC2 (Q1 KA gate) | DAC1 (Q2 brake gate) | Pass condition |
| --- | --- | --- | --- | --- |
| 1 | V_BLACK = 8V (throttle) | 3.3V | 0V | Transparent |
| 2 | V_BLACK = 0V (brake), pot full CCW | 0V | ~1.5V DC | Soft brake |
| 3 | Brake active, pot full CW | 0V | ~2.5V DC | Maximum braking |
| 4 | Brake active, pot mid travel | 0V | ~2.0V DC | Variable resistance — smooth DC throughout |
| 5 | Any pot position, full throttle | 0V | 0V or release ramp | No injection at any pot position |

## 9.4 Scope verification sequence — Mode C (brushless keep-alive)

Set the mode toggle to C. In mode C, Q1 floor is active when V_BLACK drops below the saved keep-alive DAC setpoint. If no value has been saved, Q1 stays off until a capture is performed.

| Step | Condition | DAC2 (Q1 KA gate) | DAC1 (Q2 brake gate) | Pass condition |
| --- | --- | --- | --- | --- |
| 1 | V_BLACK = 8V (throttle), value saved | 3.3V (Q1 off — V_BLACK above floor) | 0V | Normal throttle — Q1 suppressed |
| 2 | V_BLACK drops to 0.3V (brake) | Set to saved_ka_dac (Q1 floors BLACK) | ~1.5–2.5V per pot | Brake + keep-alive floor active |
| 3 | Capture button held, pot swept | Tracks pot: 3.3V at CCW, ~2.0V at CW | 0V | Q1 gate voltage live on pot — tune keep-alive |
| 4 | Capture button released | Latches to saved_ka_dac | Resumes normal | LED blinks 3×; value saved to flash |
| 5 | Remove MCU power | 3.3V (Q1 off) | ~1.5V+ via pull-up | Safe default — Q2 in brake region, Q1 off |

Step 7 (MCU power removal) is the most critical safety test across all modes. The pull-up circuit on the Q2 op-amp buffer input must drive Q2 into its braking region when the MCU is unpowered. Verify the DAC1 buffer output reaches at least DAC_BRAKE_SOFT voltage equivalent (~1.5V) with the MCU off.

## 9.5 On-track tuning

Once scope verification passes, tune the following parameters via UART SET commands. Use SAVE to persist values. Changes take effect immediately without reflashing.

| Parameter | Command | Default | Tuning guidance |
| --- | --- | --- | --- |
| Minimum brake (soft stop) | SET BRAKE_SOFT <dac> | 1860 | Raise DAC value for harder minimum brake. Verify feel with motor under light load. ~1860 corresponds to ~1.5V gate = onset of Q2 conduction. |
| Brake entry threshold | SET BRAKE_ENTER <mV> | 500mV | Reduce if brake is slow to engage. Increase if brake activates at light throttle settings. |
| Brake exit threshold | SET BRAKE_EXIT <mV> | 1500mV | Raise if brake state persists too long. Lower if it releases too early. |
| Ramp-in time | Capture button (A/B) | 0ms | Hold button, rotate pot CW to increase ramp time. CCW = instant. Typical: 50–150ms for wing cars. |
| Trail brake release | SET RELEASE_MS_A/B/C <ms> | 0ms | 0 = instant snap-off. 50–200ms for progressive trail. Watch RELEASE state in UART telemetry. |
| Keep-alive floor (mode C) | Capture button (C) | — | Hold button, rotate pot CW until eCom powers up reliably, then back off slightly. |
| Anti-brake ceiling (A/B) | SET KA_MAX <dac> | 2480 | Reduce if maximum anti-brake causes instability. Increase for more aggressive anti-brake. |
| Latvian brake threshold | SET LATVIAN_DVDT <mV/ms> | 0 (off) | Enable with 1000. Watch telemetry for false triggers during normal braking. |

# 10. Three-Mode Design — Rationale and Behaviour

## 10.1 The braking problem

When a slot car driver releases the throttle trigger fully, the hand controller shorts the track terminals. The motor, still spinning, becomes a generator and dumps its stored kinetic energy back into that short. This energy pulse is brief — typically a few microseconds — but can reach 12V at 20A.

The amount of resistance placed across the motor terminals determines how hard and how quickly the car decelerates. A dead short extracts energy as fast as possible, stopping the car in the shortest distance. A higher resistance bleeds energy more slowly, giving a softer, more progressive stop. The original module design offered a single variable brake from hard short to approximately 8Ω effective resistance.

Two further requirements drove the three-mode architecture:

- Some brushed motors — particularly high-Kv types used in Group 12 competition — stop so violently under a dead short that the car becomes unstable. These motors benefit from a small forward voltage applied against the braking force (antibrake) to slow the deceleration rate.

- Brushless motors controlled by an eCom (electronic commutator) require the eCom’s onboard MCU to remain powered at all times. If the supply voltage to the eCom drops to zero during braking, the MCU resets and the brushless motor loses commutation lock. A continuously present keep-alive voltage prevents this.

Positive anti-brake (modes A/B) and eCom keep-alive floor (mode C) are mutually exclusive by motor type and use different mechanisms. Anti-brake actively injects voltage to counteract braking force in brushed motors. Keep-alive floors the BLACK_to_track voltage to prevent the eCom MCU browning out — it does not supply the eCom, which is powered by the controller. The mode toggle enforces this separation in hardware.

## 10.2 Mode A — Brushed motor with antibrake

Mode A is intended for brushed motors where abrupt braking is a problem. The centre-detent potentiometer divides the control range into two regions separated by a deadband that always maps to dead short (maximum braking).

| Pot position | Q2 (brake FET) | Q1 (antibrake FET) | Effective behaviour |
| --- | --- | --- | --- |
| Full CCW | PWM — BRAKE_CCR_SOFT | OFF | ~8Ω across motor — softest brake |
| CCW → centre | PWM ramping harder | OFF | Brake resistance decreases toward dead short |
| Centre ± deadband | Fully ON (CCR=0) | OFF | Dead short — maximum braking |
| Past centre → CW | OFF | PWM ramping up | Antibrake 0→3V — counters braking force |
| Full CW | OFF | PWM — KA_CCR_MAX | ~2V injected — maximum positive anti-brake |
| Wiper open (throttle) | OFF | OFF | Module transparent — no intervention |

The centre detent sits at dead short rather than freewheel. This is a deliberate design choice: the physical click of the detent gives the driver a tactile reference for maximum braking. Rotating clockwise past the detent enters antibrake territory. The deadband (±150 ADC counts) ensures the transition is clean and cannot oscillate between modes at the detent position.

## 10.3 Mode B — Brushed motor, brake only

Mode B is for brushed motors that do not need antibrake. The full pot range maps to brake resistance only, giving the driver maximum resolution over the braking force from minimum to maximum.

| Pot position | Q2 (brake FET) | Q1 | Effective behaviour |
| --- | --- | --- | --- |
| Full CCW | PWM — BRAKE_CCR_SOFT | Always OFF | ~8Ω — softest brake |
| Mid travel | PWM interpolated | Always OFF | Variable resistance |
| Full CW | Fully ON (CCR=0) | Always OFF | Dead short — hardest brake |
| Wiper open | OFF | OFF | Module transparent |

There is no deadband or centre region in mode B. The entire pot sweep is available for dialling in brake feel, which is useful for drivers who want fine control over a narrow resistance range rather than a split brake/antibrake range.

## 10.4 Mode C — Brushless motor with eCom keep-alive

Mode C is for brushless motors driven by an eCom. The fundamental difference from modes A and B is that Q1 is never fully off — it continuously injects a small saved voltage onto the track terminals to keep the eCom MCU powered.

| Condition | Q2 (brake FET) | Q1 (keep-alive FET) | Effective behaviour |
| --- | --- | --- | --- |
| Wiper open, normal running | OFF | ON at saved CCR | Keep-alive voltage always on track; throttle passes through normally |
| V_BLACK < 500mV, pot CCW | PWM — BRAKE_CCR_SOFT | ON — floors V_BLACK above eCom minimum | ~8Ω brake, Q1 prevents brownout |
| V_BLACK < 500mV, pot CW | Fully ON (CCR=0) | ON — floors V_BLACK | Dead short, Q1 prevents brownout |
| Capture button held | OFF | ON, pot-controlled | User dials keep-alive voltage; brake suspended |
| Capture button released | Resumes normal | Saved CCR written to flash | LED blinks 3× to confirm save |

The keep-alive voltage is saved permanently to the last flash page (page 31, address 0x08007C00) and restored on every power-up. This means the eCom-specific voltage only needs to be dialled in once per motor/controller combination.

Hardware caution: when Q2 is fully on (dead short) and Q1 is simultaneously injecting keep-alive voltage, the track terminals are held near 0V by Q2 while Q1 is driving current into that near-zero impedance from the 12.4V rail. A Schottky diode in series with Q1’s output (between Q1 drain and the track terminal) is essential in mode C. The diode blocks reverse current through Q1 during the brake pulse and protects Q1 from the voltage spike when Q2 turns off. This diode is not needed in modes A or B where Q2 and Q1 are never on simultaneously.

## 10.5 Keep-alive voltage capture procedure

The correct keep-alive voltage for a given eCom is the lowest voltage at which the eCom MCU stays powered without commanding the brushless motor to spin. This voltage is specific to each eCom design and must be found empirically.

Capture procedure:

- Set the mode toggle to position C (brushless keep-alive).

- Place the car on a bench with the motor free to spin but not under load.

- Connect track power. The eCom will initially be unpowered if no keep-alive voltage has been saved yet.

- Hold the capture button. The brake is suspended and the pot now directly controls Q1 output voltage.

- Slowly rotate the pot clockwise from full CCW (0V) until the eCom powers up and its status LED illuminates (or other power-on indication).

- Continue rotating very slowly until the brushless motor just begins to twitch or spin. Note this position.

- Rotate slightly counter-clockwise until the motor stops but the eCom remains powered. This is the target voltage.

- Release the capture button. The current voltage is written to flash and the status LED blinks three times to confirm the save.

- Verify: cycle power. The eCom should power up immediately on track voltage application without the capture button being held.

If the eCom requires a higher voltage than expected, check whether the eCom has an internal LDO regulator or uses the track voltage directly. Some eCom designs will start spinning the motor at any positive voltage above their UVLO threshold, in which case a small series resistor from Q1 to the track may be needed to limit inrush current during the capture procedure.

# 11. FET Behaviour Reference

## 11.0 Linear mode operation — why and how

Both Q1 and Q2 are operated in their linear (triode) region rather than as switches. This is the most important aspect of the output stage design and deserves a full explanation.

## 11.0.1 The three regions of a MOSFET

A MOSFET has three operating regions depending on the relationship between Vgs (gate-source voltage) and Vds (drain-source voltage):

| Region | Condition | Behaviour | Used for |
| --- | --- | --- | --- |
| Cut-off | Vgs < Vth | No drain current. FET is an open circuit. | FET off — passthrough mode |
| Saturation | Vgs > Vth, Vds > Vgs − Vth | Drain current controlled by Vgs only, independent of Vds. FET acts as a current source. | Switching applications (digital) |
| Linear (triode) | Vgs > Vth, Vds < Vgs − Vth | Drain current controlled by both Vgs and Vds. FET acts as a voltage-controlled resistor. | Variable resistance — this design |

In the linear region, the FET’s drain-source resistance (Rds) is a function of Vgs:

Rds(on) ∝ 1 / (Vgs − Vth)²  (approximate, simplified)

For IRLB3034 (Q2):

Vth typical = 1.5V

Vgs = 1.5V → Rds ∞ (FET just at threshold, barely conducting)

Vgs = 2.0V → Rds ≈ several ohms

Vgs = 2.5V → Rds ≈ 5mΩ (near rated Rds(on))

The useful brake control range is Vgs ≈ 1.5V to 2.5V — a 600mV window.

The 12-bit DAC gives 4096 steps over 0–3.3V = 0.8mV per step.

750 steps span the 600mV window: resolution ≪ any mechanical pot.

## 11.0.2 Why linear mode, not PWM switching

The alternative to linear mode is PWM switching: the FET is switched fully on and off at high frequency (20kHz), and the motor inductance integrates the switching to produce an effective resistance. PWM was the original design but was replaced with linear mode for three reasons:

- Brushless eCom compatibility. The eCom MCU monitors its supply voltage on the braid. A 20kHz switching brake FET superimposes a large ripple voltage on the braid that confuses the eCom ADC and can cause it to misfire or reset. A linear FET presents a smooth, continuous resistance with no switching noise.

- EMC regulations. High-frequency switching circuits must comply with CISPR 32 / EN 55032 conducted and radiated emission limits. A slot car module with a 20kHz PWM output on a track that acts as an antenna is a genuine compliance problem for a commercial product. Linear control has no switching frequency and no harmonic content.

- Resolution. PWM effective resistance depends on motor inductance, which varies between motors and with speed. The relationship is L / (duty × T), which changes non-linearly. DAC-driven linear control is directly proportional to gate voltage, which is independent of motor parameters.

## 11.0.3 The gate drive challenge in linear mode

Operating a FET in its linear region requires holding its gate at a precise intermediate voltage. This is fundamentally different from switching mode where the gate is driven hard to either rail. Two specific challenges arise:

- Stability. A FET in linear mode has a negative thermal coefficient at high current: as the junction heats, Rds(on) increases, reducing current, which reduces heating. This is self-stabilising for the FET. However the op-amp buffer driving the gate must be stable with a capacitive gate load. A small gate resistor (R3/R4, 10Ω) provides isolation between the op-amp output and the gate capacitance.

- Safe default. In switching mode, a pull-up to HIGH forces the FET hard on. In linear mode, the pull-up must land within the linear region (Vgs between Vth and full enhancement), not just force the gate high. For Q2 (IRLB3034, Vth ≈ 1.5–2V), the pull-up target is approximately 1.8–2.0V — enough to put Q2 into moderate conduction (light-to-medium braking) without full short. This is still a safe condition: the car brakes, but not violently. The exact pull-up voltage depends on the op-amp supply rail and resistor divider used.

The safe-default pull-up for Q2 intentionally targets the lower end of the linear region rather than maximum conduction. This avoids a violent dead-short on every power-cycle while still ensuring the car cannot coast freely without MCU power. For competition use, a harder default can be set by adjusting the pull-up divider.

## 11.1 Q1 — Anti-brake / keep-alive FET (P-channel, high-side)

Q1 (IRF9Z34N, P-channel) connects the WHITE rail to the BLACK_to_track node in a high-side configuration. Its source is at WHITE rail voltage (12–16V). Gate is driven by the DAC2 op-amp buffer.

P-channel convention: gate must be driven BELOW source to turn on. Vgs is negative when on.

| DAC2 output | Vgate | Vgs (source at 12V) | Q1 state | Effect |
| --- | --- | --- | --- | --- |
| DAC_KA_OFF = 4095 | 3.3V | 3.3 − 12 = −8.7V | Near fully on | Wait — see note below |
| DAC_KA_OFF = 4095 | 3.3V | Vgs(th) for IRF9Z34N = −2 to −4V | FET actually ON here | Q1 is driven ON when DAC is at 3.3V |

Important clarification: with source at 12V and gate at 3.3V, Vgs = −8.7V which is well past the IRF9Z34N threshold of −2V to −4V. This means DAC = 3.3V actually turns Q1 FULLY ON, not off. To turn Q1 OFF, the gate must be at approximately the same voltage as the source, i.e. ~12V. The op-amp buffer must be powered from a supply above the WHITE rail or use a bootstrap circuit to achieve this. This is an open design item — the gate drive circuit for Q1 requires careful attention. DAC_KA_OFF in the current firmware represents the intention; the hardware implementation must ensure the op-amp can actually swing the gate to near V_WHITE to turn Q1 off.

Q1 functions in two roles:

- Modes A/B (positive anti-brake): when the driver releases the trigger into the anti-brake zone (pot CW past centre), Q1 injects a small forward voltage from WHITE onto the track terminal, opposing the braking force. DAC2 set to DAC_KA_MAX (~2.0V gate) gives approximately 2V effective injection.

- Mode C (keep-alive floor): Q1 activates when V_BLACK would otherwise drop below the saved keep-alive threshold. It prevents the eCom MCU from seeing a brownout during braking. It does not supply the eCom — it only clamps the floor.

## 11.2 Q2 — Brake FET (N-channel, low-side)

Q2 (IRLB3034, N-channel) connects BLACK_to_track to RED/GND in a low-side configuration. Gate is driven by the DAC1 op-amp buffer.

N-channel convention: gate must be driven ABOVE source (at GND) to turn on. Vgs is positive when on.

| DAC1 output | Vgate | Q2 state | Rds approx | Motor sees |
| --- | --- | --- | --- | --- |
| DAC_BRAKE_OFF = 0 | 0V | Off (Vgs < Vth) | Open circuit | No brake |
| DAC_BRAKE_SOFT = 1860 | ~1.5V | Linear region onset | Several Ω | Soft brake (~8Ω target) |
| Intermediate | ~2.0V | Mid linear region | ~0.5Ω | Medium brake |
| DAC_BRAKE_HARD = 3095 | ~2.5V | Deep linear / saturation boundary | ~5mΩ | Near dead short |
| MCU off, pull-up active | ~1.8V (design target) | Lower linear region | Light-medium brake | Safe default |

The DAC_BRAKE_SOFT value of 1860 is a starting point corresponding to approximately 1.5V gate voltage. The actual effective resistance at this setting depends on the specific motor inductance and speed. Calibrate on the bench using SET BRAKE_SOFT <dac> over UART while observing braking feel with the motor under load.

## 11.3 Simultaneous Q1 and Q2 conduction (Mode C only)

In modes A and B, Q1 and Q2 are never on simultaneously. In mode C they can be: Q1 maintains the keep-alive floor while Q2 brakes. This creates an electrical conflict during hard braking:

When Q2 pulls BLACK_to_track to near 0V (hard brake), Q1 is simultaneously driving current from WHITE (12–16V) into this near-zero impedance. Without protection, this current flows backward through Q1’s body diode at high repetitive peak current.

I_peak ≈ V_WHITE / Rds_Q1 ≈ 12V / 0.35Ω ≈ 34A instantaneous

Occurring at the keep-alive DAC update rate — effectively continuous during hard braking

D4 (MBRS360 or SS54) in series with Q1’s output blocks this reverse current path. D4 is essential in mode C. It can be omitted on mode A/B-only boards.

# 12. Updated Pin Map — STM32G051K6U6 (QFN-32)

## 12.1 Rationale for package change

The original design used the STM32G041J6M6 in an SOIC-8 package with only 6 usable GPIO pins. Adding the three-mode toggle switch (two pins), capture button (one ADC pin), and status LED (one pin) required four additional signals, making the SOIC-8 unsuitable.

The STM32G051K6U6 in QFN-32 was selected as the replacement. It uses the identical Cortex-M0+ core, the same 32K flash and 8K RAM, and runs the same firmware binary without modification. The QFN-32 package provides 20 usable GPIO pins, giving substantial headroom for future additions. The package is compact (5×5mm) and solderable with hot-air rework equipment.

## 12.2 Full pin assignment

| Pin | Signal | Type | Function |
| --- | --- | --- | --- |
| PA0 | Rail sense | ADC_IN0 analog | 12.4V rail ÷ voltage divider → ≤3.3V |
| PA1 | Wiper strip | GPIO input, pull-up | Active LOW — trigger wiper contact |
| PA2 | Toggle bit 0 | GPIO input, pull-up | LOW = mode B or invalid; decoded with PA4 |
| PA3 | Pot wiper | ADC_IN3 analog | 10k pot, 0–3.3V |
| PA4 | Toggle bit 1 | GPIO input, pull-up | LOW = mode C or invalid; decoded with PA2 |
| PA5 | Capture button | ADC_IN5 analog | 10k pull-up; button → GND via 1k. Unpressed ~4095, pressed ~372 |
| PA6 | Brake PWM | TIM3_CH1 AF1 | Q2 gate via GD2. INVERTED polarity. |
| PA7 | Keep-alive PWM | TIM3_CH2 AF1 | Q1 gate via GD1. Normal polarity. |
| PB6 | Status LED | GPIO output | Blinks 3× on successful keep-alive save to flash |

## 12.3 Toggle switch decoding

| PA2 state | PA4 state | Mode selected | Notes |
| --- | --- | --- | --- |
| HIGH (unpressed) | HIGH (unpressed) | A — Brushed + antibrake | Both pins floating high |
| LOW (pressed) | HIGH (unpressed) | B — Brushed, brake only | Bit 0 asserted |
| HIGH (unpressed) | LOW (pressed) | C — Brushless keep-alive | Bit 1 asserted |
| LOW (pressed) | LOW (pressed) | B (safe default) | Invalid combination — firmware defaults to mode B |

## 12.4 Capture button ADC circuit

The capture button uses a single ADC pin rather than a dedicated GPIO. A 10kΩ pull-up resistor connects PA5 to 3.3V. The capture button connects PA5 to GND via a 1kΩ series resistor. This produces two well-separated ADC voltage levels:

| Button state | V_PA5 | ADC count (12-bit) | Firmware interpretation |
| --- | --- | --- | --- |
| Unpressed | 3.3V | ~4095 | Not pressed |
| Pressed | 1k/(10k+1k) × 3.3V = 0.3V | ~372 | Pressed |
| Detection threshold | Midpoint ~1.8V | ~2200 | BTN_PRESSED_THRESHOLD constant |

Using ADC for the button rather than a GPIO has two advantages: the same ADC scan that reads the pot and rail also reads the button, with no additional interrupt or polling overhead; and the analogue threshold can be adjusted in firmware (BTN_PRESSED_THRESHOLD) to accommodate component tolerance without a hardware change.

# 13. Brake Ramp-In and Industry Terminology

## 13.1 Industry context

In slot car racing, three distinct braking techniques are in common use, and the terminology is sometimes used inconsistently by different manufacturers. The AART module implements all three, and the mode names have been updated to match industry convention:

| AART mode | Industry term | Description |
| --- | --- | --- |
| Mode B | Reduced braking | Variable resistance inserted across motor terminals during braking. No voltage applied. Most common implementation — some manufacturers incorrectly call this anti-brake. |
| Mode A (brake side) | Reduced braking | Same as mode B on the CCW half of the pot. Dead short at the centre detent. |
| Mode A (anti-brake side) | Positive anti-brake | Small forward voltage applied to motor when trigger released. Creates gentle forward torque opposing the braking force. Used on advanced controllers from Difalco, Third Eye Technology, and others. |
| Mode C | eCom keep-alive | Continuous low voltage maintains eCom MCU power regardless of throttle state. Specific to brushless motor installations. |

The AART module uses direct track power (WHITE rail) for all voltage injection. No stored energy (capacitors, batteries, supercapacitors) is used. This satisfies the direct track power requirement in current BSCRA and ISRA controller regulations, where energy storage devices that can continue driving the car after trigger release are typically prohibited. Verify against the specific class rules for the event.

## 13.2 Why positive anti-brake helps

When a high-downforce wing car releases the trigger at speed, a dead short across the motor creates an abrupt deceleration that can destabilise the car. The effect is described by racers as the car ‘tripping over’ or ‘snapping’ under braking.

Positive anti-brake applies a small forward torque (typically 0.2–2V) when the trigger is released. This:

- Reduces peak deceleration rate

- Keeps the rear tyres loaded during corner entry

- Makes the car feel ‘longer’ and smoother through fast sweepers

- Reduces deslotting risk under heavy braking

The effect is most noticeable with powerful motors on high-downforce cars and tracks with long flowing corners. On tight technical tracks with heavy braking zones, less anti-brake (or none) is usually preferred for corner precision.

## 13.3 Brake ramp-in

The brake ramp-in feature delays the onset of full braking after the trigger is released, creating an ‘active coast’ effect. Instead of applying the full pot-selected resistance immediately, the module ramps the brake CCR exponentially from zero toward the target value.

### 13.3.1 Exponential ramp shape

Each millisecond tick, the CCR moves a fraction of the remaining distance toward the target:

ccr += (target − ccr) × alpha / 256   (each 1ms tick)

alpha = 256  →  0ms  (immediate, one step)

alpha = 128  →  ~10ms to reach 95% of target

alpha = 26   →  ~50ms to reach 95% of target

alpha = 3    →  ~200ms to reach 95% of target

The exponential shape gives a fast initial brake onset (preventing the car running wide) followed by a gentler tail-off, which avoids the snap-stop that upsets high-downforce cars. This matches the feel of the ‘active coast’ circuits found in advanced commercial controllers.

### 13.3.2 Setting the ramp time

The ramp time is set using the capture button in modes A and B:

- Set the mode toggle to A or B.

- Hold the capture button. The brake is suspended.

- Rotate the pot: CCW = 0ms (immediate braking), CW = 200ms (slowest ramp).

- Release the button. The ramp time is saved to flash and the LED blinks 3×.

- The saved value persists across power cycles.

Starting point recommendations:

| Car type | Suggested ramp time | Rationale |
| --- | --- | --- |
| High-downforce wing car, powerful motor | 80–150ms | Prevents snap-stop on corner entry |
| Standard wing car | 30–80ms | Moderate onset softening |
| Pan car / inline | 0–30ms | Precise braking needed for tight corners |
| Brushless (mode C) | Not applicable | Ramp-in disabled in mode C — ESC needs prompt braking |

### 13.3.3 Interaction with positive anti-brake

In mode A, the ramp-in applies only to the brake (CCW) side of the pot. The positive anti-brake (CW side) takes effect immediately when the trigger releases and the pot is in the anti-brake zone. There is no ramp on the anti-brake injection — a gradual onset of forward torque would feel sluggish and is not beneficial.

## 13.4 Flash storage layout

The flash save record at page 31 (address 0x08007C00) now stores both the keep-alive CCR and the ramp time in a single 64-bit write:

Offset 0x00: uint32_t magic    = 0xAA270001

Offset 0x04: uint16_t ka_ccr   (bits 15:0  of upper word)

uint16_t ramp_ms  (bits 31:16 of upper word)

Full 64-bit record:

[63:48] = ramp_ms  (0–200)

[47:32] = ka_ccr   (0–400)

[31:0]  = magic    (0xAA270001)

If the magic word is not present (erased flash = all 0xFF), both values default to zero (immediate braking, keep-alive off). The capture procedure only needs to be performed once per motor/controller combination.

# 13B. Trail Brake Release

## 13B.1 Concept

In real motor racing, trail braking is the technique of maintaining partial brake pressure while beginning to apply throttle at the corner entry. The brakes are not released all at once; they are faded out smoothly as the car transitions from braking to acceleration. This gives the driver finer control over weight transfer and cornering balance.

The module implements this by providing a configurable linear release ramp on brake exit. When the driver picks up the throttle (V_BLACK rises above the brake exit threshold), instead of snapping the brake FET immediately to off, the DAC gate voltage ramps linearly from the current brake value down to zero over a configured number of milliseconds.

## 13B.2 Entry vs release — different shapes by design

The brake entry ramp is exponential: fast initial bite, gentler tail. This matches what a driver needs on entry — the car must stop overrunning immediately, but the final hard phase can be progressive.

The brake release ramp is linear: a steady, constant rate of fade. This is a deliberate design choice. A linear release is predictable — the driver can feel exactly how quickly the brakes are fading and modulate their throttle application accordingly. An exponential release would feel unpredictable: slow at first, then suddenly snapping off.

| Phase | Shape | Rate | Why |
| --- | --- | --- | --- |
| Brake entry (ramp-in) | Exponential | Fast initial onset, decaying tail | Immediate grip on brake application; gentle progression prevents snap |
| Brake release (trail) | Linear | Constant rate per millisecond | Predictable fade the driver can feel and trust during corner entry |

## 13B.3 Configuration

Three independent release times, one per mode, set via UART and saved to flash:

| Command | Default | Range | Typical starting points |
| --- | --- | --- | --- |
| SET RELEASE_MS_A <ms> | 0 (instant) | 0–500ms | Pan cars: 50–100ms. Wing cars: 100–200ms. |
| SET RELEASE_MS_B <ms> | 0 (instant) | 0–500ms | Same as mode A guidance. |
| SET RELEASE_MS_C <ms> | 0 (instant) | 0–500ms | Brushless: 30–80ms. Longer values may cause eCom issues. |

## 13B.4 Full throttle snap override

The trail release ramp is overridden automatically if V_BLACK rises sharply above the brake exit threshold plus a snap margin (RELEASE_SNAP_MV = 500mV). This detects a hard, aggressive throttle application and snaps the brake immediately to off — the driver does not want trailing brake resistance when asking for full power.

The snap override ensures that trail braking only operates in the transition zone — light throttle while still bleeding off speed. It cannot interfere with a normal full-throttle acceleration out of a corner.

## 13B.5 Telemetry

The state field in UART telemetry shows RELEASE while the trail ramp is active, and PASS when it completes. This allows the release duration to be tuned on the bench by watching the state column while simulating throttle pickup.

# 14B. Latvian Brake — Emergency Stop on Track Call

## 14B.1 Concept and use case

During a race a marshal may cut track power in an emergency (car accident, deslot into dangerous position, track obstruction). When this happens the track PSU output collapses instantly. A conventional braking module that relies on the hand controller’s output to detect the brake condition will not respond — the controller’s output collapses along with everything else.

The Latvian brake solves this by monitoring the WHITE rail (always-live PSU output) for a rapid voltage collapse. If WHITE drops faster than a configured threshold, the module fires an immediate dead short across the motor, independent of the driver’s trigger position, mode selection, or pot setting.

The name ‘Latvian brake’ refers to the origin of this concept in the Latvian eCom (Remora) development. In the original eCom implementation the brake is handled inside the ESC. In this module it is implemented externally, making it available for any motor type without modification to the ESC.

## 14B.2 Trigger — WHITE rail dV/dt

The module samples WHITE every 1ms. The Latvian brake fires when the drop in WHITE between consecutive 1ms samples exceeds the configured threshold:

drop_mv = white_prev_mv - white_mv   (this tick - last tick)

if drop_mv >= latvian_dvdt_mv_per_ms: fire Latvian brake

Example: threshold = 1000 mV/ms

Normal throttle release: WHITE drops ~50mV/ms -> no trigger

Track call / PSU cut:    WHITE drops 3000mV/ms -> TRIGGER

This distinguishes a gradual throttle release (WHITE falls slowly as the controller PWMs down) from a sudden PSU cut (WHITE collapses in one or two ADC samples). The threshold is configurable via UART and disabled by default.

## 14B.3 Behaviour when active

| Condition | Q2 (brake) | Q1 (keep-alive) | State reported |
| --- | --- | --- | --- |
| Latvian brake fires | Immediately BRAKE_CCR_HARD (dead short) | KA_CCR_OFF (no injection) | STATE_LATVIAN_BRAKE |
| Latvian brake active | Held at dead short — all normal mode logic bypassed | Off — WHITE rail too unstable to support injection | STATE_LATVIAN_BRAKE |
| Release: WHITE and BLACK both > 1500mV | Returns to normal mode logic | Returns to normal | Normal state resumes |

The Latvian brake takes absolute priority over all other logic including capture mode, ramp-in, and keep-alive. It cannot be overridden by pot position or mode selection while active. The hardware pull-up on GD2 maintains the dead short even if the MCU browns out before the software fires — the hardware is the last line of defence.

## 14B.4 Configuration via UART

| Command | Example | Effect |
| --- | --- | --- |
| SET LATVIAN_DVDT <mV/ms> | SET LATVIAN_DVDT 1000 | Enable Latvian brake at 1V/ms threshold. Fires if WHITE drops > 1V in any 1ms sample. |
| SET LATVIAN_DVDT 0 | SET LATVIAN_DVDT 0 | Disable Latvian brake entirely (default). |
| GET | GET | Shows current latvian_dvdt value with ENABLED/DISABLED status. |

Recommended threshold values:

| Threshold | Value | Rationale |
| --- | --- | --- |
| Conservative | 500 mV/ms | Fires on any drop faster than ~24ms to zero from 12V. May trigger on aggressive PWM controllers. |
| Recommended | 1000 mV/ms | Fires when WHITE drops > 1V per millisecond. Track call typically drops 8–16V in 2–3ms. Normal throttle release is < 100mV/ms. |
| Aggressive | 2000 mV/ms | Only fires on near-instantaneous collapse. Safe from false triggers but slower response. |
| Disabled | 0 | Default. Latvian brake inactive. Normal braking and hardware safe-default still apply. |

Changes via SET LATVIAN_DVDT are runtime-only and revert on power cycle. The feature is intentionally not saved to flash — it should be consciously enabled at each session until the threshold is proven reliable for a specific track and PSU combination. Future hardware revisions may add a dedicated enable switch.

## 14B.5 Telemetry

The UART CSV output includes a latvian_active column (last field) which is 1 while the Latvian brake is latched and 0 otherwise. The state field shows LATVIAN when active. This allows the threshold to be tuned on the bench by watching the telemetry while simulating a track call (rapidly removing PSU power).

# 14. UART Debug Interface and Field Flashing

## 14.1 Overview

Three complementary debug and flashing mechanisms are provided:

| Method | Connector | Requires | Use case |
| --- | --- | --- | --- |
| SWD debug | J10 (4-pin 2.54mm) | ST-Link V2/V3 + OpenOCD | Full hardware debug: breakpoints, watchpoints, live variable inspection. Primary development tool. |
| UART telemetry | J11 (3-pin 2.54mm) | USB-UART adapter (3.3V logic) | Live CSV telemetry every 100ms. Runtime threshold adjustment without reflashing. Primary bench tuning tool. |
| ROM UART bootloader | J11 + J12 + J13 | USB-UART adapter + stm32flash | Field flashing without ST-Link. No custom bootloader code required — built into STM32 ROM. |

## 14.2 SWD debug header (J10)

| Pin | Signal | MCU pin | Notes |
| --- | --- | --- | --- |
| 1 | SWDIO | PA13 (AF0) | Bidirectional data |
| 2 | SWDCLK | PA14 (AF0) | Clock from programmer |
| 3 | GND | — | Common ground |
| 4 | +3.3V | — | Optional — powers target from programmer |

PA13 and PA14 default to SWD after reset — no firmware configuration needed. The debug.sh script starts OpenOCD and connects GDB, breaking at main(). Useful commands during tuning:

p s_ctx                   // full BrakeCtx_t struct

p s_ctx.black_mv          // live BLACK terminal voltage in mV

p s_ctx.brake_ccr         // current brake CCR value

watch s_ctx.brake_active  // break when brake state changes

p uart_debug_get_params() // current runtime threshold values

## 14.3 UART telemetry header (J11)

| Pin | Signal | Connect to |
| --- | --- | --- |
| 1 | TX (PA9, USART1 AF1) | RX of USB-UART adapter |
| 2 | RX (PA10, USART1 AF1) | TX of USB-UART adapter |
| 3 | GND | GND of adapter |

Settings: 115200 8N1, 3.3V logic. Use a 3.3V adapter — do not connect 5V UART directly to PA9/PA10. On macOS: screen /dev/tty.usbserial-XXXX 115200. On Linux: minicom -D /dev/ttyUSB0 -b 115200.

Telemetry output format (every 100ms):

#tick,state,mode,white_mv,black_mv,brake_ccr,ka_ccr,pot_raw,ramp_ms,brake_active

12340,BRAKE,A,12400,0,1850,0,512,80,1

12440,RAMP,A,12400,0,2100,0,512,80,1

12540,PASS,A,12400,8200,3199,0,512,80,0

## 14.4 Runtime commands

Send commands terminated with CR or LF. Changes take effect immediately but revert on power cycle — they are not saved to flash. Use the capture button to permanently save ka_ccr and ramp_ms.

| Command | Example | Effect |
| --- | --- | --- |
| SET BRAKE_ENTER <mV> | SET BRAKE_ENTER 400 | Brake entry threshold. Reduce if brake is slow to engage. |
| SET BRAKE_EXIT <mV> | SET BRAKE_EXIT 1200 | Brake exit threshold. Raise if brake drops out at low throttle. |
| SET BRAKE_SOFT <ccr> | SET BRAKE_SOFT 2400 | Minimum brake resistance CCR. Lower = more braking at CCW. |
| SET KA_MAX <ccr> | SET KA_MAX 300 | Maximum anti-brake / keep-alive CCR (~voltage). |
| GET | GET | Print current runtime parameter values. |
| SAVE | SAVE | Write ALL current runtime parameters to flash (brake_enter, brake_exit, brake_soft, ka_ccr, ramp_ms, latvian_dvdt). Survives power cycle. LED blinks 3× to confirm. Note: ka_ccr and ramp_ms are also saved by the capture button. |
| SET RELEASE_MS_A <ms> | SET RELEASE_MS_A 150 | Trail brake release time for mode A (0–500ms). 0=instant snap-off. Configures how long the brake linearly fades after the driver picks up throttle. |
| SET RELEASE_MS_B <ms> | SET RELEASE_MS_B 100 | Trail brake release time for mode B (0–500ms). |
| SET RELEASE_MS_C <ms> | SET RELEASE_MS_C 50 | Trail brake release time for mode C brushless (0–500ms). |
| GET ID | GET ID | Returns firmware identity string. Format: #ID: AART-BRAKE-v8-STM32G051 |
| HELP | HELP | Print command list. |

## 14.5 BOOT0 and NRST buttons

| Ref | Signal | Function |
| --- | --- | --- |
| J12 | BOOT0 (PB8) | Momentary button pulls PB8/BOOT0 to 3.3V via 10k. Normally LOW via internal pull-down. |
| J13 | NRST | Momentary button pulls NRST to GND. Resets the MCU. |

## 14.6 ROM UART bootloader procedure

The STM32G041 contains a factory-programmed ROM bootloader accessible via PA9/PA10. No custom bootloader code is required or installed.

- Hold the BOOT0 button (J12).

- Press and release NRST (J13) while holding BOOT0.

- Release BOOT0. MCU is now in ROM UART bootloader on PA9/PA10 at 115200 baud.

- Flash: stm32flash -w build/aart_brake.bin -v -g 0x0 /dev/ttyUSB0

- Or use STM32CubeProgrammer in UART mode at the same port.

- Press NRST once to boot the new firmware. BOOT0 button not needed.

stm32flash for macOS: brew install stm32flash. It handles the ROM bootloader handshake automatically. The same J11 UART adapter used for telemetry is used for flashing — no additional hardware needed for field updates.

# 15. Potentiometer Resolution and Control Precision

## 15.1 The signal chain

The potentiometer produces a voltage between 0 and 3.3V across its 300° of mechanical travel. The STM32G041 ADC converts this to a 12-bit value (0–4095 counts). The firmware maps this ADC count to a TIM3 CCR value (0–3199) which controls the PWM duty cycle on Q2 or Q1. The PWM duty cycle determines the effective resistance or injection voltage seen at the motor terminals.

| Stage | Range | Steps | Resolution per degree |
| --- | --- | --- | --- |
| Pot rotation | 0–300° mechanical | — | — |
| ADC output | 0–4095 counts | 4096 | ~13.7 counts/° |
| TIM3 CCR (brake region) | 0–2720 (BRAKE_CCR_HARD to BRAKE_CCR_SOFT) | 2720 | ~9.1 CCR/° in brake half of mode A |
| TIM3 CCR (anti-brake region) | 0–400 (KA_CCR_MAX) | 400 | ~2.7 CCR/° across anti-brake half |
| Effective resistance | 0Ω (dead short) to ~8Ω | Continuous | ~0.05–0.5Ω/° (non-linear) |
| Anti-brake voltage | 0–2V (at 12.4V rail) | ~31mV per CCR step | ~1–2mV/° |

## 15.2 Effective resistance — how PWM braking works

The brake FET (Q2) does not present a fixed resistance. It switches on and off at 20kHz. The motor’s inductance integrates this switching, so the motor ‘sees’ an effective resistance that depends on the duty cycle and the motor’s electrical time constant:

R_eff ≈ L / (duty_off × T_period)

Where:

L         = motor winding inductance (~50–200µH typical for Group 12)

duty_off  = fraction of period when Q2 is OFF (= CCR / ARR)

T_period  = 1 / f_PWM = 50µs at 20kHz

Example at BRAKE_CCR_SOFT = 2720, L = 100µH, 20kHz:

duty_off  = 2720 / 3200 = 0.850

R_eff     = 100µH / (0.850 × 50µs) = 2.35Ω

The ‘~8Ω’ figure cited throughout this document is an approximation assuming a typical motor inductance. The real value depends heavily on the specific motor. At 100µH the CCR_SOFT setting gives ~2–3Ω; at 50µH it halves to ~1–1.5Ω. The BRAKE_SOFT constant should be tuned on the bench with the actual motor using the UART telemetry to observe the braking feel. The effective resistance is also speed-dependent — back-EMF modifies the current that actually flows.

## 15.3 Resistance resolution across the brake range

The resistance curve is highly non-linear. Near the CCW (soft brake) end, each degree of pot rotation changes resistance by a large amount because the inductance-based formula is sensitive to duty_off near 1.0. Near the CW (dead short) end, each degree changes resistance very little because the curve flattens asymptotically toward 0Ω.

| Pot position | Approx CCR | Effective R (100µH, 20kHz) | R change per degree |
| --- | --- | --- | --- |
| Full CCW (min brake) | 2720 | ~2.4Ω | — |
| 25% travel | 2040 | ~1.0Ω | ~0.06Ω/° |
| 50% travel (mode B centre) | 1360 | ~0.4Ω | ~0.03Ω/° |
| 75% travel | 680 | ~0.12Ω | ~0.01Ω/° |
| Full CW (dead short) | 0 | 0Ω | ≈ 0 (hard limit) |

This non-linearity is actually beneficial for tuning. The first half of pot travel gives coarse adjustment across the most useful braking range. The second half compresses the remaining variation toward dead short, where fine control matters less — a dead short is a dead short.

## 15.4 Anti-brake voltage resolution

The anti-brake region maps the CW half of the pot (past the mode A centre deadband) to Q1 CCR values 0–400. At a 12.4V rail this gives 0–2V in steps of approximately:

V_step = V_RAIL / (ARR + 1) = 12.4 / 3200 = 3.875 mV per CCR count

Pot resolution in anti-brake zone:

ADC counts over anti-brake region: ~2048 - 150 = ~1898 counts

CCR steps over anti-brake region: 400

Pot degrees for anti-brake: ~150°

CCR per degree: 400 / 150 = 2.67 CCR/°

Voltage per degree: 2.67 × 3.875mV = ~10mV/°

Voltage per ADC count: 3.875mV × (400 / 1898) = ~0.8mV/count

At 10mV per degree of rotation, a hand steady to ±2° gives ±20mV anti-brake voltage resolution. This is adequate for the intended use — the goal is to find a rough setting that changes car feel, not to hit a specific millivolt target.

## 15.5 The binding constraint: PWM quantisation

The fundamental limit on anti-brake precision is not the pot or the ADC — it is the PWM resolution. With ARR=3199 the smallest voltage step is one CCR count = V_RAIL/3200 ≈ 3.9mV at 12.4V. No amount of pot quality or ADC oversampling can improve this without changing the PWM frequency or resolution.

If finer anti-brake control is needed, reduce KA_CCR_MAX via UART. Setting KA_MAX to 100 instead of 400 restricts the full pot travel to a 0–0.5V range, giving four times finer resolution for the same mechanical movement:

| KA_CCR_MAX setting | Voltage range | V per CCR step | V per degree of pot |
| --- | --- | --- | --- |
| 400 (default) | 0–2.0V at 12.4V | ~3.9mV | ~10mV |
| 200 | 0–1.0V | ~3.9mV | ~5mV |
| 100 | 0–0.5V | ~3.9mV | ~2.5mV |
| 50 | 0–0.25V | ~3.9mV | ~1.25mV |

The voltage per CCR step stays constant (it depends only on rail voltage and ARR). Reducing KA_CCR_MAX compresses the pot range, effectively zooming in on the lower end of the voltage scale. Use SET KA_MAX via UART to experiment with this on the bench before saving.

## 15.6 Pot specification recommendations

A standard 10kΩ linear pot is entirely adequate for this application. The ADC has far more resolution than the mechanical pot requires. However, the following properties do matter:

- Linearity: ±0.5–1% is sufficient. The non-linear resistance curve dominates at the soft-brake end regardless of pot linearity.

- Mechanical backlash: keep to ≤1° for anti-brake use. At 10mV/°, 1° of backlash causes ±10mV hysteresis in anti-brake voltage — acceptable.

- Centre detent: strongly recommended for Mode A. The detent gives the driver a tactile reference for the dead-short / anti-brake transition point. Without it, the transition is invisible and the driver cannot reliably find the boundary.

- Resistance value: 10kΩ is ideal. Lower values increase current draw from the 3.3V reference; higher values increase source impedance for the ADC (above ~50kΩ the 39.5-cycle sampling time may introduce errors). 10k is the sweet spot.

- Panel type vs servo: a panel-mount pot with a knob is more practical than a servo pot for this application. The driver adjusts between runs, not in real time — a knob with a scale marker is more useful than a multi-turn precision pot.

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | May 2026 | AART | Initial design document |
| 2.0 | May 2026 | AART | Added sections 8–9: libopencm3 firmware, Docker toolchain, G0-specific API notes, pre-commissioning checklist and scope verification sequence |
| 3.0 | May 2026 | AART | Added sections 10–12: three-mode design rationale, full FET behaviour reference with polarity explanation, keep-alive capture procedure, simultaneous Q2/Q1 conduction analysis, updated pin map for STM32G051K6U6 QFN-32 |
| 4.0 | May 2026 | AART | Updated section 9: scope verification tables split per mode, correct pot directions, expanded tuning table |
| 5.0 | Jun 2026 | AART | Major revision: sections 1–4 rewritten for Rev 4 architecture. Wiper strip removed; brake detection via BLACK terminal voltage sense with hysteresis (enter <500mV, exit >1500mV). Keep-alive suppressed when controller above KA setpoint. BLACK sense divider R10/R11 (22k/4k7) added. ADC scan extended to 4 channels. Rail voltage updated to 12–16V. Banana plug connections documented. |
| 6.0 | Jun 2026 | AART | Rev 5 firmware: exponential brake ramp-in added (0–200ms, saved to flash). Industry terminology adopted. Capture button dual-function. Flash record extended to 64-bit. BSCRA/ISRA legality note added. |
| 7.0 | Jun 2026 | AART | Comprehensive document audit: stale HAL-era constants replaced; title page updated; sections 4–6 corrected for V_BLACK brake detection; section 5 rewritten as Peripheral Configuration; section 7 open items refreshed; new section 7B Key Design Decisions added. |
| 8.0 | Jun 2026 | AART | Power and thermal sanity check applied. Component changes: D1 P6KE18A → P6KE18CA (bidirectional); D2/D3 SS34 → SS310 (100V); D4 SS34 → MBRS360; F1 25A → 500mA–1A WHITE rail only; R8 15k → 18k. New section 3.5 Power and Thermal Analysis added. |
| 9.0 | Jun 2026 | AART | Removed stale CubeMX Setup Checklist entirely. Duplicate stale bench test table removed. |
| 9.1 | Jun 2026 | AART | Section 4.3 corrected: HAL_ADC_ConvCpltCallback replaced with dma1_channel1_isr(); two-element buffer updated to four channels. |
| 10.0 | Jun 2026 | AART | Added UART debug and field flashing (section 14): USART1 on PA9/PA10 at 115200 8N1, CSV telemetry every 100ms, runtime SET/GET command interface. BOOT0 button J12 (PB8) and NRST button J13 added for ROM UART bootloader entry without ST-Link. SWD header J10 updated with +3.3V pin 4. KiCad schematic updated with all three debug/flash circuits. Firmware: new uart_debug.c/h, brake_get_ctx() API, runtime-adjustable thresholds. |
| 11.0 | Jun 2026 | AART | Added Latvian brake (section 14B): WHITE rail dV/dt monitoring for emergency dead short on track call / PSU cut. Configurable threshold via SET LATVIAN_DVDT <mV/ms>, 0=disabled (default). Bypasses all normal mode logic, ramp-in, and keep-alive when active. STATE_LATVIAN_BRAKE added to telemetry. Firmware Rev 6. |
| 11.1 | Jun 2026 | AART | Added UART SAVE command: persists all runtime SET parameters (brake thresholds, brake_soft, latvian_dvdt, ka_ccr, ramp_ms) to flash in one operation. Flash record extended to 24 bytes (three 64-bit double-words). Magic bumped to 0xAA270002. uart_debug_load_params() restores all values on boot. LED blinks 3× to confirm SAVE. |
| 12.0 | Jun 2026 | AART | Added section 15: Potentiometer Resolution and Control Precision. Full signal chain, PWM quantisation as binding constraint, resistance non-linearity, anti-brake voltage resolution, KA_CCR_MAX scaling for finer control, pot specification recommendations. |