AART Slot Car Braking Module

Design Document

Target MCU: STM32G041K6U6 (QFN-32) | Firmware: Rev 5 | libopencm3

Revision 1.0  |  AART — Adrian & Richard’s Technologies

# 1. Overview

This document describes the hardware design and firmware architecture for the AART slot car braking module. The module sits in-line between a conventional hand controller and the track, providing variable braking, freewheel, and antibrake functions under the control of a single centre-detent potentiometer.

## 1.1 Purpose

Some Group 12 motors with high Kv ratings stop too abruptly when a conventional dead-short brake is applied. The module addresses this by providing:

- Variable braking — from a hard short (full stop) to a soft ~8Ω effective resistance

- Freewheel — both FETs off, motor coasts freely

- Antibrake — a small forward voltage (0–2V) injected against the stopping force to smooth deceleration

## 1.2 System position

The module connects via 4mm banana plugs to the three standard track sockets and exports matching sockets for the hand controller to plug into. It is fully transparent during normal running and takes over braking when the controller shorts the track.

| Banana colour | Signal | Description |
| --- | --- | --- |
| WHITE | +12–16V always-live rail | From track PSU. Can deliver 40–90A. Powers the module and Q2 injection. |
| BLACK | Track positive (+ve to car) | Controlled by hand controller. Monitored by module to detect brake condition. |
| RED | Common negative | Left braid return. Module and track ground reference. |

The module presents identical WHITE/BLACK/RED sockets on its output side for the hand controller to plug into. During normal running, BLACK passes through unmodified. When the hand controller shorts BLACK→RED (brake condition), the module detects this by voltage sense on the BLACK terminal and applies its own controlled braking.

The WHITE rail is always live at 12–16VDC regardless of throttle position. The hand controller varies how much of this rail reaches the BLACK output. The module draws its MCU supply from WHITE via the LDO and uses WHITE as the source for Q2 injection voltage.

# 2. Operating Modes

The module has three operating modes selected by a 3-position toggle switch. Within each mode the brake is triggered by voltage sense on the BLACK terminal rather than a mechanical wiper strip contact, making the module compatible with any hand controller.

## 2.1 Brake detection — voltage sense with hysteresis

The BLACK terminal voltage is continuously monitored via an ADC input. A hysteresis band prevents chatter at low throttle settings near zero:

- V_BLACK < 500mV → enter brake state (controller has shorted BLACK→RED)

- V_BLACK > 1500mV → exit brake state (controller delivering power)

- 500mV ≤ V_BLACK ≤ 1500mV → hold current state

This approach works with any hand controller regardless of whether it has an accessible wiper strip contact, and correctly handles partial braking (controllers that do not short fully to zero).

## 2.2 Mode A — Brushed motor with antibrake

| V_BLACK | Pot position | Q1 (brake) | Q2 (antibrake) | Behaviour |
| --- | --- | --- | --- | --- |
| > 1500mV | Any | OFF | OFF | Pass-through — module transparent |
| < 500mV | Full CCW | PWM — ~8Ω | OFF | Softest brake |
| < 500mV | CCW → centre | PWM ramping | OFF | Brake resistance decreasing |
| < 500mV | Centre ±deadband | Fully ON | OFF | Dead short — max braking |
| < 500mV | Centre → CW | OFF | PWM 0→2V | Antibrake injection ramping up |
| < 500mV | Full CW | OFF | PWM — ~2V | Maximum antibrake |

## 2.3 Mode B — Brushed motor, brake only

| V_BLACK | Pot position | Q1 (brake) | Q2 | Behaviour |
| --- | --- | --- | --- | --- |
| > 1500mV | Any | OFF | Always OFF | Pass-through |
| < 500mV | Full CCW | PWM — ~8Ω | Always OFF | Softest brake |
| < 500mV | Full CW | Fully ON | Always OFF | Dead short — max braking |

## 2.4 Mode C — Brushless motor, eCom keep-alive

Q2 provides a small continuous voltage to keep the eCom MCU powered. It is suppressed when the hand controller is delivering power above the keep-alive setpoint to prevent Q2 fighting the controller output.

| V_BLACK vs keep-alive | Wiper state | Q1 (brake) | Q2 (keep-alive) | Behaviour |
| --- | --- | --- | --- | --- |
| V_BLACK > KA setpoint | Not braking | OFF | OFF (suppressed) | Controller powering eCom — Q2 not needed |
| V_BLACK ≤ KA setpoint | Not braking | OFF | ON at saved CCR | Keep-alive active — eCom powered by module |
| V_BLACK < 500mV | Braking | PWM per pot | ON at saved CCR | Brake + keep-alive in parallel |
| Capture button held | Either | OFF | Pot-controlled | User dials keep-alive voltage |

During a hard brake (Q1 dead short), V_BLACK collapses to near 0V — well below the keep-alive setpoint — so Q2 runs during braking. The Schottky diode D4 in series with Q2’s output is essential in mode C to block reverse current through Q2 when Q1 holds the track near 0V.

# 3. Hardware Design

## 3.1 Component list

| Ref | Component | Value / Part | Notes |
| --- | --- | --- | --- |
| U1 | MCU | STM32G041K6U6 | QFN-32, 3.3V, 64MHz |
| U2 | LDO regulator | AMS1117-3.3 | WHITE rail → 3.3V, SOT-223 |
| GD1 | Gate driver | UCC27524 | Brake FET channel, SOT-23-5 |
| GD2 | Gate driver | UCC27524 | Keep-alive FET channel, SOT-23-5 |
| Q1 | Brake FET | IRLB3034 N-ch | 195A, 1.4mΩ Rds(on), D2PAK |
| Q2 | Keep-alive FET | IRF9Z34N P-ch | High-side switch, 18A, D2PAK |
| D1 | TVS diode | P6KE18CA (bidirectional) | Rail transient clamp, 18V, SMB. CA suffix essential — bidirectional variant clamps both polarity transients from inductive switching. |
| D2 | Schottky | SS310 | Q1 catch diode, 3A/100V SMB. 100V reverse voltage gives margin over SS34’s 40V on a 16V rail with inductive spikes. |
| D3 | Schottky | SS310 | Q2 catch diode, 3A/100V SMB. Same upgrade as D2. |
| D4 | Schottky | SS34 | Mode C series output diode — ESSENTIAL |
| D5 | LED red | 0805 | Status indicator PB6 |
| F1 | Polyfuse | 500mA–1A, 1812 | WHITE rail supply protection for MCU/LDO only. Must NOT be in the BLACK (track) path — a polyfuse cannot handle 40A motor startup and will trip from thermal lag. The BLACK path carries full motor current; PSU overcurrent protection covers that rail. |
| R1 | 10kΩ | 0402 | NRST pull-up |
| R2 | 10kΩ | 0402 | GD1 input pull-up (safe default brake) |
| R3 | 10Ω | 0402 | Q1 gate resistor |
| R4 | 10Ω | 0402 | Q2 gate resistor |
| R5 | 10kΩ | 0402 | Pot top-end to 3.3V |
| R6 | 10kΩ | 0402 | Capture button pull-up PA5 |
| R7 | 1kΩ | 0402 | Capture button series to GND |
| R8 | 15kΩ | 0402 | WHITE sense top divider (PA0) |
| R9 | 4k7Ω | 0402 | WHITE sense bottom divider (PA0) |
| R10 | 22kΩ | 0402 | BLACK sense top divider (PA1) |
| R11 | 4k7Ω | 0402 | BLACK sense bottom divider (PA1) |
| R12 | 330Ω | 0402 | Status LED current limit |
| C1–C4 | Decoupling | 0805/0402 | 100nF + 10μF per LDO rail |
| C5–C7 | Decoupling | 0402 | 100nF VDD, 100nF VDDA, 4.7μF VDDA |
| J1–J3 | 4mm banana sockets | Track side | WHITE / BLACK / RED to track |
| J4–J6 | 4mm banana sockets | Controller side | WHITE / BLACK / RED from hand controller |
| J7 | 3-pin 2.54mm | Pot connector | 3.3V / wiper / GND |
| J8 | 3-pin 2.54mm | Mode toggle | Bit0 / Bit1 / GND |
| J9 | 2-pin 2.54mm | Capture button | Momentary pushbutton |

## 3.2 Pin assignments — STM32G041K6U6 QFN-32

| Pin | Signal | Type | Function |
| --- | --- | --- | --- |
| PA0 | WHITE sense | ADC_IN0 analog | WHITE rail ÷ R8/R9 divider. V_PA0 = V_WHITE × 0.2386. Safe to 16V. |
| PA1 | BLACK sense | ADC_IN1 analog | BLACK track terminal ÷ R10/R11 divider. V_PA1 = V_BLACK × 0.1760. Brake detection. |
| PA2 | Toggle bit 0 | GPIO input, pull-up | LOW = mode B or invalid. Decoded with PA4. |
| PA3 | Pot wiper | ADC_IN3 analog | 10k pot, 0–3.3V. |
| PA4 | Toggle bit 1 | GPIO input, pull-up | LOW = mode C or invalid. Decoded with PA2. |
| PA5 | Capture button | ADC_IN5 analog | 10k pull-up; button → GND via 1k. ~4095 unpressed, ~372 pressed. |
| PA6 | Brake PWM | TIM3_CH1 AF1 | Q1 gate via GD1. INVERTED polarity. CCR=0 → dead short. |
| PA7 | Keep-alive PWM | TIM3_CH2 AF1 | Q2 gate via GD2. Normal polarity. CCR=0 → Q2 off. |
| PB6 | Status LED | GPIO output | Blinks 3× on keep-alive save to flash. |

## 3.3 Voltage dividers

Both sense inputs use the same bottom resistor (4k7Ω) for component commonality. The top resistor differs to accommodate the different maximum voltages:

| Signal | R_top | R_bot | Ratio | V_pin at 16V rail | Safe? |
| --- | --- | --- | --- | --- | --- |
| WHITE (PA0) | 15kΩ | 4k7Ω | 0.2386 | 3.82V — marginally over | Add 12k top for 2.96V (see note) |
| WHITE (PA0) revised | 18kΩ | 4k7Ω | 0.2073 | 3.32V | ✓ safe to 3.3V rail |
| BLACK (PA1) | 22kΩ | 4k7Ω | 0.1760 | 2.82V | ✓ safe to 3.3V rail |

If the track PSU can deliver up to 16V, use R8=18kΩ (not 15kΩ) for the WHITE sense divider. Update WHITE_DIV_NUM/DEN in brake_module.h to 47/234 accordingly. The original 15k value was sized for 12.4V supplies; at 16V it produces 3.82V on PA0 which exceeds the 3.3V ADC reference.

## 3.5 Power and thermal analysis

The following analysis confirms component ratings against the worst-case electrical demands of the application. Track PSU: 12.4V nominal, 16V maximum. Motor startup: 40A for a few milliseconds. Braking back-EMF: 20A+ momentarily. PSU cannot absorb reverse current.

| Component | Worst-case demand | Rating | Verdict | Action taken |
| --- | --- | --- | --- | --- |
| Q1 IRLB3034 (brake FET) | 20A braking current continuous during brake P = I² × Rds = 400 × 0.0014 = 0.56W ΔT = 35°C above ambient (no heatsink) | 195A, 40V, 175°C Tj Rds(on) 1.4mΩ Rth_ja 62°C/W | ✓ Comfortable | No change. Motor startup current (40A) does not flow through Q1 — Q1 is OFF during throttle. Copper pour under D2PAK pad recommended. |
| Q2 IRF9Z34N (keep-alive FET) | Mode C keep-alive: 5–50mA (eCom load) — negligible Simultaneous Q1+Q2: up to 46A instantaneous through Q2 body diode if D4 absent | 18A, 55V, 175°C Tj Rds(on) 350mΩ @ Vgs=−3.3V | ✓ OK (with D4) | D4 mandatory in mode C. Without D4, body diode of Q2 carries 46A repetitively at 20kHz — not rated for this. D4 blocks this current path. |
| D1 TVS (P6KE18CA) | Motor back-EMF adds to rail: 16V + 12V peak = 28V transient Energy: <<600W × 1ms pulse capacity | 600W peak (10/1000µs), Vc=25.2V @ 1A | ✓ OK | Changed to bidirectional CA variant. Unidirectional P6KE18A only clamps one polarity; inductive switching generates both. |
| D2/D3 SS310 (catch diodes) | Q1 turn-off spike: L × dI/dt = 100µH × 20A/1µs = 2000V unclamped D2 clamps at Vf + Vdc ≈16.4V, energy = 20mJ | 3A avg, 100V, same SMB footprint as SS34 | ✓ Upgraded | Changed from SS34 (40V) to SS310 (100V). D2 is essential — without it Q1’s 40V Vds is instantly exceeded on turn-off. 40V reverse rating left no margin on 16V rail. |
| D4 MBRS360 (mode C series diode) | Repetitive ~46A pulses at 20kHz during mode C hard braking (Q2 ON-time into near-shorted track via Q1) | 3A avg, 60V, 150A peak (single pulse) Same SMB footprint | ✓ Upgraded | Changed from SS34 (80A peak, marginal under repetitive 20kHz stress) to MBRS360 (60V, same SMB footprint). SS54 (5A, 150A peak) is equally suitable. D4 position is the most electrically stressed diode in the design. |
| F1 polyfuse (WHITE rail) | MCU + LDO + gate drivers ≈ 35mA total on WHITE rail No motor current on WHITE rail during braking or throttle | Resettable fuse, now 500mA–1A | ✓ Corrected | Downrated from 25A to 500mA–1A. F1 protects the MCU/LDO supply on WHITE only. It must NOT be in the BLACK (track) path which carries 40A motor startup and 20A braking current — a polyfuse would trip. |
| U2 AMS1117-3.3 (LDO) | Load: ~35mA P = (16 − 3.3) × 0.035 = 0.44W at 16V rail ΔT = 0.44 × 62 = 27°C (SOT-223, no heatsink) | 1A, 125°C Tj, Rth_ja 62°C/W | ✓ OK | Tj = 52°C at 25°C ambient, 97°C at 70°C ambient. Both within 125°C limit. Solder SOT-223 tab to copper pour for better thermal performance. |

Motor startup current of 40A does not flow through any module component during normal operation. Q1 and Q2 are both OFF when the hand controller is delivering power (V_BLACK > 1500mV). The module is fully passive during throttle. Only during braking does current flow through Q1, and only then does the back-EMF need to be managed.

## 3.4 Safe default behaviour

On power loss or MCU fault, the brake FET (Q1) must default to ON (hard brake). This is achieved by:

- A 10kΩ pull-up resistor (R2) on the GD1 input. When the MCU output goes high-impedance (reset/power-off), R2 holds GD1 input HIGH → driver output HIGH → Q1 gate HIGH → FET on → hard brake.

- TIM3_CH1 is configured with INVERTED polarity so CCR=0 produces a HIGH output during normal operation, consistent with the pull-up logic.

- brake_force_safe() writes directly to TIM3 CCR registers and is called from hard_fault_handler and nmi_handler.

# 4. Firmware Architecture

## 4.1 File structure

| File | Description |
| --- | --- |
| Core/Inc/brake_module.h | Constants, pin map, types, public API |
| Core/Src/brake_module.c | Control logic, ADC/DMA, PWM, flash storage |
| Core/Src/main.c | Top-level: SysTick ISR, fault handlers, main loop |

## 4.2 Timing and scheduling

The module uses a polling architecture. brake_tick() is called from the main loop as fast as possible and uses a millisecond counter (incremented by SysTick ISR) for internal pacing. ADC conversions run via DMA so the CPU never blocks waiting for a result.

## 4.3 ADC configuration

ADC1 is configured in scan mode with four channels in sequence:

- Rank 1 — PA0 (CH0): WHITE rail voltage sense

- Rank 2 — PA1 (CH1): BLACK track voltage sense

- Rank 3 — PA3 (CH3): pot wiper

- Rank 4 — PA5 (CH5): capture button ADC

DMA transfers all four results to a circular buffer in a single transfer. The libopencm3 ISR dma1_channel1_isr() sets a flag; the main tick reads the buffer on the next 1ms iteration. Sampling time is 39.5 ADC cycles — adequate for the 10kΩ pot and voltage divider source impedances.

## 4.4 PWM configuration

TIM3 runs at 20kHz (above the audio band, good for logic-level FETs):

SYSCLK = 64 MHz, PSC = 0, ARR = 3199

Frequency = 64,000,000 / (0+1) / (3199+1) = 20,000 Hz

CH1 polarity: INVERTED (OCPolarity_LOW)

CCR = 0    -> output HIGH -> brake FET ON  (hard brake)

CCR = 3199 -> output LOW  -> brake FET OFF (no brake)

CH2 polarity: NORMAL (P-channel Q2 — gate LOW = FET on)

CCR = 0   -> output LOW  -> Q2 OFF (no injection)

CCR = 400 -> PWM 400/3200 = 12.5% duty -> ~2V at 16V rail

Note: effective voltage scales with WHITE rail voltage

Recalculate KA_CCR_MAX if PSU voltage changes: CCR = V_target/V_rail x 3200

## 4.5 Key constants

| Constant | Default | Meaning |
| --- | --- | --- |
| ADC_CENTRE | 2048 | Midpoint of 12-bit ADC range |
| ADC_DEADBAND | 150 | ±3.7% of range — freewheel zone around centre |
| BRAKE_DUTY_HARD | 0 | CCR for full hard short (FET fully on) |
| BRAKE_DUTY_SOFT | 2720 | CCR for ~8Ω effective resistance at pot minimum |
| ANTI_DUTY_MAX | 516 | CCR for ~2V antibrake injection at 12.4V rail |
| WIPER_DEBOUNCE_MS | 10 | Wiper strip debounce time in milliseconds |
| ADC_RAIL_DIVIDER | 0.2386 | Voltage divider ratio R3/(R2+R3) for rail sense |
| RAIL_UNDERVOLTAGE_MV | 5000 | Below this rail voltage — force safe brake state |

## 4.6 Control flow

Each 1ms tick executes the following sequence:

- Update ADC readings from DMA buffer if conversion complete

- Check rail voltage — force safe brake if below RAIL_UNDERVOLTAGE_MV

- Read debounced wiper strip state (PA1)

- If wiper open: set MODE_PASSTHROUGH, both FETs off, return

- Read pot ADC value and compare against deadband thresholds

- Below deadband: compute brake duty, update TIM3_CH1, set antibrake to 0

- Above deadband: compute antibrake duty, update TIM3_CH2, set brake to full off

- Within deadband: both FETs off — MODE_FREEWHEEL

# 6. Bench Testing

## 6.1 Brake detection verification

The brake trigger is V_BLACK based. To simulate brake conditions on the bench, apply a variable voltage source to the BLACK input terminal and verify PA6/PA7 respond correctly:

| V_BLACK applied | Expected PA6 (brake) | Expected PA7 (Q2) | State |
| --- | --- | --- | --- |
| 0V (short to RED) | HIGH or PWM per mode/pot | Per mode | Brake active |
| 0.3V | HIGH or PWM | Per mode | Brake active (below 500mV threshold) |
| 0.5–1.5V | Holds previous state | Holds previous state | Hysteresis zone — no change |
| 2V | LOW (if was in passthrough) | Per mode C / OFF for A/B | Brake released |
| 8V (throttle mid) | LOW | OFF (A/B) or OFF if suppressed (C) | Pass-through |
| 16V (full throttle) | LOW | OFF | Pass-through, Q2 suppressed in mode C |

## 6.2 Power budget

At 20A motor current with IRLB3034 (Rds(on) = 1.4mΩ typical):

P_Q1 = I² × Rds(on) = 400 × 0.0014 = 0.56W (hard short, full current)

P_Q2 = I_keepalive² × Rds(on)  — negligible (eCom load is milliamps)

P_LDO = (V_WHITE − 3.3V) × I_MCU = (16 − 3.3) × 0.02 = 0.25W typ

Well within D2PAK ratings for Q1. The LDO dissipation increases at higher rail voltages — at 16V consider a small heatsink on U2 or a pre-regulator resistor to drop some voltage before the LDO.

# 7. Open Items and Known Limitations

| Item | Status | Action required |
| --- | --- | --- |
| BRAKE_CCR_SOFT calibration | Default 2720 (~8Ω) is calculated, not measured | Verify on bench with motor and 20A supply. Adjust until minimum brake feel is acceptable. |
| WHITE sense divider | R8=18k/R9=4k7 gives 3.31V at 16V — 1mV over nominal but within ADC abs max (3.6V) | Resolved: 18k fitted. For additional margin use 20k (3.15V at 16V). Update WHITE_DIV_DEN to 247 if changed. |
| Q2 gate drive at high rail voltage | UCC27524 supply is 3.3V; Q2 gate swing is only 0–3.3V with P-ch IRF9Z34N Vgs(th)=-3V | Verify Q2 turns on fully at 3.3V gate drive. Consider BSS84 (Vgs(th)=-0.8V to -1.5V) for guaranteed enhancement at 3.3V. |
| LDO dissipation at 16V | AMS1117 dissipates ~0.25W at 16V rail with MCU load | Add heatsink tab or copper pour under SOT-223 tab for sustained high-rail use. |
| Ramp-in reset on mode change | If mode toggle is changed mid-ramp, ramp state is not cleared | Low priority — add mode_prev tracking and reset ramp on mode change if needed. |
| KiCad schematic | Schematic has wires and labels but component symbols not yet placed | Place symbols from KiCad libraries using BOM CSV as reference. |

# 7B. Key Design Decisions

This section records the significant architectural choices made during development, the alternatives considered, and the rationale for each decision. This is intended to prevent the same questions being relitigated in future revisions.

| Decision | Choice made | Alternatives considered | Rationale |
| --- | --- | --- | --- |
| Brake trigger detection | Voltage sense on BLACK terminal with hysteresis (500mV enter, 1500mV exit) | Mechanical wiper strip contact from hand controller; optoisolator across controller output | Voltage sense works with any hand controller regardless of mechanical design. Hysteresis prevents chatter at low throttle. Wiper strip requires physical access to the controller internals and would not detect partial-short braking. Optoisolator adds complexity and a component. |
| MCU package | STM32G041K6U6 QFN-32 | STM32G041J6M6 SOIC-8 (original design) | SOIC-8 has only 6 usable GPIO. Adding BLACK sense (PA1), mode toggle (PA2, PA4) and capture button (PA5) requires 4 additional pins unavailable in SOIC-8. QFN-32 uses same die, same firmware, same flash/RAM. QFN is solderable with hot air. |
| Firmware library | libopencm3 (no ST HAL, no CubeMX) | ST HAL with CubeMX code generation | Eliminates proprietary toolchain dependency. Entire build chain is open source and reproducible in Docker. Resulting binary is ~4.6KB vs ~20-40KB typical HAL. No ST account or licence required. CubeMX generates fragile boilerplate that is hard to version-control. |
| Q1 brake FET polarity | INVERTED (CCR=0 → FET on = dead short) | Normal polarity (CCR=ARR → FET on) | With inverted polarity, the safe failure mode (MCU unpowered, gate floating) defaults to brake ON via the pull-up resistor on GD1 input. With normal polarity, loss of MCU results in no braking — the car runs freely into a corner. Safety requirement drives this choice. |
| Q2 FET type | P-channel IRF9Z34N (high-side) | N-channel with bootstrap gate driver (high-side); N-channel low-side with reversed injection | P-channel requires no bootstrap circuit. Gate driven directly by UCC27524 from 3.3V rail. N-channel high-side requires gate voltage above drain (above rail) — needs charge pump or bootstrap, adding components and complexity. Low-side injection would require motor re-wiring. |
| Keep-alive suppression | Q2 off when V_BLACK > ka_ccr_to_mv(saved_ka_ccr, white_mv) | Q2 always on (rely on D4 Schottky); fixed voltage threshold | Always-on Q2 fights the hand controller output when V_BLACK is above the keep-alive setpoint. This causes unnecessary current through both Q2 and the Schottky. Dynamic threshold using actual rail voltage scales correctly across different PSU voltages. Fixed threshold would be wrong at non-nominal rail voltages. |
| Brake ramp shape | Exponential (geometric decay: ccr += error × alpha/256 per ms) | Linear ramp; S-curve (two-phase coast then ramp) | Exponential gives fast initial onset (prevents car running wide) followed by a gentler tail (avoids snap-stop). Matches the feel of commercial active-coast controllers. Linear ramp has abrupt onset at entry and at completion. S-curve was considered but adds code complexity with marginal feel benefit. |
| Capture button implementation | Single ADC pin (10k pull-up, 1k series to GND via button) | Dedicated GPIO; long-press vs short-press on GPIO | ADC approach uses the same scan that reads the pot and rail — no extra interrupt or polling path. ADC threshold is adjustable in firmware without hardware change. Shares one of the available ADC channels (PA5/CH5). GPIO would waste a pin; long-press adds UX complexity. |
| Flash storage | Single 64-bit double-word at page 31 (magic + ka_ccr + ramp_ms) | Multiple 32-bit writes; EEPROM emulation library | G0 flash requires 64-bit aligned writes. One record fits both saved values. Page 31 reserved by linker (LENGTH=31K) so firmware can never overwrite it. EEPROM emulation adds significant code size and complexity for a two-variable save requirement. |
| Anti-brake voltage ceiling | 2V (KA_CCR_MAX=400 at 16V rail) | 3V; adjustable via second capture | 2V is at the top of the practical range described in commercial controllers (0.2–2V typical). 3V risks spinning the motor rather than providing gentle anti-brake. A hardcoded ceiling with field capture for the specific value needed gives safety and flexibility. |

# 8. Firmware Build — libopencm3 and Docker Toolchain

## 8.1 Library choice

The firmware was written against libopencm3 rather than the ST HAL. This eliminates the CubeMX dependency, keeps the entire toolchain open source, and produces a significantly smaller binary. The compiled debug image is 4,616 bytes of text — approximately 14% of the available 32K flash.

libopencm3 is included as a git submodule and pre-built inside the Docker image for the STM32G0 family only, keeping the image lean. No ST account, no proprietary downloads, no generated code blobs are required to build the project from scratch.

## 8.2 G0-specific differences from other STM32 families

Several libopencm3 API differences apply specifically to the STM32G0 and are worth documenting here for anyone porting from F-series examples:

- DMAMUX — The G0 DMA controller routes requests through a DMAMUX peripheral. Unlike F0/F1 where the DMA channel-to-peripheral mapping is fixed in hardware, the G0 requires an explicit write to DMAMUX1_C0CR to connect ADC1 to DMA1 Channel 1. In the firmware this is done with a direct MMIO32 write to address 0x40020800 (DMAMUX1 base, channel 0 register).

- ADC scan mode — adc_enable_scan_mode() does not exist for G0. Scan is implicit when the sequence length is greater than one. Set the sequence with adc_set_regular_sequence() and the hardware scans automatically.

- ADC resolution symbol — ADC_RESOLUTION_12BIT (F-series) becomes ADC_CFGR1_RES_12_BIT on G0.

- ADC sampling time symbol — ADC_SMPR_SMP_39DOT5CYC becomes ADC_SMPR_SMPx_039DOT5CYC on G0.

- ADC DMA continuous — adc_set_dma_continue() becomes adc_enable_dma_circular_mode() on G0.

- Timer reset — timer_reset() is not available for G0 timers. Use rcc_periph_reset_pulse(RST_TIM3) instead.

- Flash wait states — The G0 rcc_clock_scale struct takes a plain integer (2) for flash_waitstates, not the FLASH_ACR_LATENCY_2WS macro.

- Linker script — libopencm3 dropped all per-device .ld files in November 2020 in favour of a script generator. The project provides its own stm32g041j6.ld containing the MEMORY block (32K flash at 0x08000000, 8K RAM at 0x20000000) and an INCLUDE cortex-m-generic.ld directive.

## 8.3 Docker toolchain

The build environment is fully containerised and reproducible. The Docker image is multi-stage:

| Stage | Contents | Build command |
| --- | --- | --- |
| base | Debian Bookworm slim + apt packages including OpenOCD 0.12.0 | (base of all stages) |
| toolchain | ARM GCC 13.3.rel1 (arm64 native) + libopencm3 pre-built for G0 | docker build --target toolchain -t aart-brake:toolchain . |
| cubeprog | toolchain + STM32CubeProgrammer (optional, needs ST installer) | docker build --target cubeprog -t aart-brake:full . |

OpenOCD is installed from the Debian Bookworm apt repository rather than built from source. The upstream GitHub release for v0.12.0 does not attach a source tarball (only auto-generated archives without a configure script), making source builds unreliable. The Debian package provides the same version without this problem.

## 8.4 Build workflow

All build, flash, and debug operations are available as simple commands:

# Compile (debug)

docker compose run --rm brake build

# Compile (release, optimised)

docker compose run --rm brake build release

# Flash via ST-Link (from host, recommended on macOS)

st-flash write build/aart_brake.bin 0x08000000

# Or via OpenOCD on the host

openocd -f interface/stlink.cfg -f target/stm32g0x.cfg \

-c "program build/aart_brake.elf verify reset exit"

On macOS, Docker Desktop does not reliably pass USB devices to containers. Build inside Docker, then flash from the host using st-flash (brew install stlink) or host OpenOCD (brew install openocd). The build/ directory is bind-mounted so the .elf/.bin/.hex files are available on the host immediately after a container build.

# 9. Pre-Commissioning Checklist

## 9.1 Hardware checks before first power-on

Work through these checks before applying track supply voltage for the first time:

- Confirm the MCU is the STM32G041K6U6 (QFN-32). The original SOIC-8 variant does not have enough pins for the three-mode design. The firmware binary is identical but the pin map is different.

- Verify the voltage divider resistors for PA0 (rail sense). Measure actual fitted values and update ADC_RAIL_DIVIDER_NUM and ADC_RAIL_DIVIDER_DEN in brake_module.h. Default assumes R_top=15kΩ, R_bottom=4.7kΩ giving 2.96V at 12.4V rail. A wrong value triggers spurious safe-brake events.

- Confirm the 10kΩ pull-up on the GD1 input is placed before the gate driver input pin, not on Q1’s gate. The gate driver fights any resistor placed after its output.

- Confirm Q2 is a P-channel FET (e.g. IRF9Z34N) driven from GD2. The UCC27524 is a low-side driver and cannot drive a P-channel gate correctly without level shifting. See section 11.2 for the required gate drive circuit.

- For any board used in mode C (brushless keep-alive): fit the SS34 Schottky diode in series with Q2’s output before the track terminal. Without this diode, Q2 takes the full braking energy pulse through its body diode every time Q1 fires during hard braking.

- Confirm catch/freewheeling Schottky diodes are in place across both Q1 and Q2 drain-source junctions. At 20A with an inductive motor load, omitting these will destroy the FETs on the first switching edge.

- Check the capture button circuit: 10kΩ pull-up from PA5 to 3.3V, 1kΩ series resistor from button to GND. Measure V_PA5 with button unpressed (~3.3V) and pressed (~0.3V) before connecting to the MCU.

## 9.2 Scope verification sequence — Mode A (brushed + antibrake)

Set the mode toggle to position A. With the MCU powered but no track supply connected, probe PA6 (brake) and PA7 (keep-alive/antibrake):

| Step | Condition | PA6 (TIM3_CH1) | PA7 (TIM3_CH2) | Pass condition |
| --- | --- | --- | --- | --- |
| 1 | Power on, wiper open | LOW | LOW | Module transparent — both FETs off |
| 2 | Wiper closed, pot full CCW | 20kHz PWM, ~15% high-time | LOW | Soft brake ~8Ω — FET switching, mostly off |
| 3 | Wiper closed, pot mid CCW side | 20kHz PWM, increasing high-time | LOW | Brake resistance decreasing toward short |
| 4 | Wiper closed, pot at centre detent | Permanently HIGH | LOW | Dead short — FET fully on |
| 5 | Wiper closed, pot mid CW side | LOW | 20kHz PWM, low duty | Antibrake ramping up from 0V |
| 6 | Wiper closed, pot full CW | LOW | 20kHz PWM ~24% duty | Max antibrake ~3V effective |
| 7 | Wiper open (any pot) | LOW | LOW | Module transparent again |
| 8 | Remove MCU power | HIGH via pull-up (DC) | LOW | Safe default — dead short without MCU |

## 9.3 Scope verification sequence — Mode B (brushed, brake only)

Set the mode toggle to position B:

| Step | Condition | PA6 (TIM3_CH1) | PA7 (TIM3_CH2) | Pass condition |
| --- | --- | --- | --- | --- |
| 1 | Wiper open | LOW | LOW | Module transparent |
| 2 | Wiper closed, pot full CCW | 20kHz PWM ~15% high-time | LOW | Soft brake ~8Ω |
| 3 | Wiper closed, pot full CW | Permanently HIGH | LOW | Dead short — FET fully on |
| 4 | Wiper closed, pot mid travel | 20kHz PWM, mid duty | LOW | Variable resistance — confirm linearity |
| 5 | Any pot position, wiper open | LOW | LOW | No injection at any pot position |

## 9.4 Scope verification sequence — Mode C (brushless keep-alive)

Set the mode toggle to position C. In mode C, PA7 should be active (non-zero) at all times once a keep-alive voltage has been saved. If no value has been saved yet, PA7 will be LOW until a capture is performed.

| Step | Condition | PA6 (TIM3_CH1) | PA7 (TIM3_CH2) | Pass condition |
| --- | --- | --- | --- | --- |
| 1 | Power on, wiper open, value saved | LOW | PWM at saved duty | Keep-alive always present |
| 2 | Wiper closed, pot full CCW | 20kHz PWM ~15% high-time | PWM at saved duty | Soft brake + keep-alive in parallel |
| 3 | Wiper closed, pot full CW | Permanently HIGH | PWM at saved duty | Dead short + keep-alive in parallel |
| 4 | Capture button held, pot CCW | LOW | LOW (0V) | Brake suspended, pot controls Q2 from 0V |
| 5 | Capture button held, pot CW | LOW | 20kHz PWM ~24% | Pot at max = ~3V on track |
| 6 | Capture button released | Resumes normal | PWM at new saved duty | LED blinks 3×; new value persists after power cycle |
| 7 | Remove MCU power | HIGH via pull-up | LOW | Safe default — dead short, keep-alive off |

Step 8 (MCU power removal) is the most critical safety test across all modes. The 10kΩ pull-up on GD1’s input must hold PA6 HIGH without the MCU. If PA6 stays LOW when the MCU is unpowered, the safe-default circuit is not working and the module must not be connected to a track.

## 9.5 On-track tuning

Once scope verification passes, the following constants in brake_module.h can be adjusted on the bench with a motor under load. All changes require a firmware rebuild and reflash.

| Constant | Default | Effect | Tuning guidance |
| --- | --- | --- | --- |
| BRAKE_CCR_SOFT | 2720 | Minimum braking strength (pot full CCW) | Increase for softer minimum brake; decrease for harder. Verify ~8Ω effective with a resistive load. |
| ADC_DEADBAND | 150 | Width of dead-short zone around centre in mode A (±counts) | Increase if mode A flickers between brake and antibrake at the detent; decrease for crisper transition. |
| KA_CCR_MAX | 774 | Maximum antibrake / keep-alive voltage (~3V at 12.4V) | Reduce if 3V antibrake is too aggressive or eCom starts spinning. Recalculate: CCR = (V_target / V_rail) × 3200. |
| WIPER_DEBOUNCE_MS | 10 | Wiper strip contact debounce | Increase to 20–30ms if the mode flickers on trigger release. |
| BTN_DEBOUNCE_MS | 20 | Capture button debounce | Increase if a single press triggers multiple saves (LED blinks more than 3×). |
| BTN_PRESSED_THRESHOLD | 2200 | ADC threshold for button detection | If button reads unreliably, measure actual V_PA5 pressed and unpressed and set threshold to midpoint × (4095/3.3). |
| RAIL_UNDERVOLTAGE_MV | 5000 | Below this rail voltage: force safe brake | Reduce if the module trips spuriously at low throttle settings; raise if it fails to protect on actual supply loss. |

# 10. Three-Mode Design — Rationale and Behaviour

## 10.1 The braking problem

When a slot car driver releases the throttle trigger fully, the hand controller shorts the track terminals. The motor, still spinning, becomes a generator and dumps its stored kinetic energy back into that short. This energy pulse is brief — typically a few microseconds — but can reach 12V at 20A.

The amount of resistance placed across the motor terminals determines how hard and how quickly the car decelerates. A dead short extracts energy as fast as possible, stopping the car in the shortest distance. A higher resistance bleeds energy more slowly, giving a softer, more progressive stop. The original module design offered a single variable brake from hard short to approximately 8Ω effective resistance.

Two further requirements drove the three-mode architecture:

- Some brushed motors — particularly high-Kv types used in Group 12 competition — stop so violently under a dead short that the car becomes unstable. These motors benefit from a small forward voltage applied against the braking force (antibrake) to slow the deceleration rate.

- Brushless motors controlled by an eCom (electronic commutator) require the eCom’s onboard MCU to remain powered at all times. If the supply voltage to the eCom drops to zero during braking, the MCU resets and the brushless motor loses commutation lock. A continuously present keep-alive voltage prevents this.

Antibrake and keep-alive are mutually exclusive by motor type. Antibrake injects voltage to counteract braking in a brushed motor. Keep-alive injects voltage to power the eCom MCU in a brushless system. Using antibrake with a brushless eCom would inject voltage into the commutation circuit at the wrong time, with unpredictable results. The mode toggle enforces this separation in hardware.

## 10.2 Mode A — Brushed motor with antibrake

Mode A is intended for brushed motors where abrupt braking is a problem. The centre-detent potentiometer divides the control range into two regions separated by a deadband that always maps to dead short (maximum braking).

| Pot position | Q1 (brake FET) | Q2 (antibrake FET) | Effective behaviour |
| --- | --- | --- | --- |
| Full CCW | PWM — BRAKE_CCR_SOFT | OFF | ~8Ω across motor — softest brake |
| CCW → centre | PWM ramping harder | OFF | Brake resistance decreases toward dead short |
| Centre ± deadband | Fully ON (CCR=0) | OFF | Dead short — maximum braking |
| Past centre → CW | OFF | PWM ramping up | Antibrake 0→3V — counters braking force |
| Full CW | OFF | PWM — KA_CCR_MAX | ~3V injected — maximum antibrake |
| Wiper open (throttle) | OFF | OFF | Module transparent — no intervention |

The centre detent sits at dead short rather than freewheel. This is a deliberate design choice: the physical click of the detent gives the driver a tactile reference for maximum braking. Rotating clockwise past the detent enters antibrake territory. The deadband (±150 ADC counts) ensures the transition is clean and cannot oscillate between modes at the detent position.

## 10.3 Mode B — Brushed motor, brake only

Mode B is for brushed motors that do not need antibrake. The full pot range maps to brake resistance only, giving the driver maximum resolution over the braking force from minimum to maximum.

| Pot position | Q1 (brake FET) | Q2 | Effective behaviour |
| --- | --- | --- | --- |
| Full CCW | PWM — BRAKE_CCR_SOFT | Always OFF | ~8Ω — softest brake |
| Mid travel | PWM interpolated | Always OFF | Variable resistance |
| Full CW | Fully ON (CCR=0) | Always OFF | Dead short — hardest brake |
| Wiper open | OFF | OFF | Module transparent |

There is no deadband or centre region in mode B. The entire pot sweep is available for dialling in brake feel, which is useful for drivers who want fine control over a narrow resistance range rather than a split brake/antibrake range.

## 10.4 Mode C — Brushless motor with eCom keep-alive

Mode C is for brushless motors driven by an eCom. The fundamental difference from modes A and B is that Q2 is never fully off — it continuously injects a small saved voltage onto the track terminals to keep the eCom MCU powered.

| Condition | Q1 (brake FET) | Q2 (keep-alive FET) | Effective behaviour |
| --- | --- | --- | --- |
| Wiper open, normal running | OFF | ON at saved CCR | Keep-alive voltage always on track; throttle passes through normally |
| Wiper contact, pot CCW | PWM — BRAKE_CCR_SOFT | ON at saved CCR | ~8Ω brake + keep-alive in parallel |
| Wiper contact, pot CW | Fully ON (CCR=0) | ON at saved CCR | Dead short + keep-alive in parallel |
| Capture button held | OFF | ON, pot-controlled | User dials keep-alive voltage; brake suspended |
| Capture button released | Resumes normal | Saved CCR written to flash | LED blinks 3× to confirm save |

The keep-alive voltage is saved permanently to the last flash page (page 31, address 0x08007C00) and restored on every power-up. This means the eCom-specific voltage only needs to be dialled in once per motor/controller combination.

Hardware caution: when Q1 is fully on (dead short) and Q2 is simultaneously injecting keep-alive voltage, the track terminals are held near 0V by Q1 while Q2 is driving current into that near-zero impedance from the 12.4V rail. A Schottky diode in series with Q2’s output (between Q2 drain and the track terminal) is essential in mode C. The diode blocks reverse current through Q2 during the brake pulse and protects Q2 from the voltage spike when Q1 turns off. This diode is not needed in modes A or B where Q1 and Q2 are never on simultaneously.

## 10.5 Keep-alive voltage capture procedure

The correct keep-alive voltage for a given eCom is the lowest voltage at which the eCom MCU stays powered without commanding the brushless motor to spin. This voltage is specific to each eCom design and must be found empirically.

Capture procedure:

- Set the mode toggle to position C (brushless keep-alive).

- Place the car on a bench with the motor free to spin but not under load.

- Connect track power. The eCom will initially be unpowered if no keep-alive voltage has been saved yet.

- Hold the capture button. The brake is suspended and the pot now directly controls Q2 output voltage.

- Slowly rotate the pot clockwise from full CCW (0V) until the eCom powers up and its status LED illuminates (or other power-on indication).

- Continue rotating very slowly until the brushless motor just begins to twitch or spin. Note this position.

- Rotate slightly counter-clockwise until the motor stops but the eCom remains powered. This is the target voltage.

- Release the capture button. The current voltage is written to flash and the status LED blinks three times to confirm the save.

- Verify: cycle power. The eCom should power up immediately on track voltage application without the capture button being held.

If the eCom requires a higher voltage than expected, check whether the eCom has an internal LDO regulator or uses the track voltage directly. Some eCom designs will start spinning the motor at any positive voltage above their UVLO threshold, in which case a small series resistor from Q2 to the track may be needed to limit inrush current during the capture procedure.

# 11. FET Behaviour Reference

## 11.1 Q1 — Brake FET (N-channel, low-side)

Q1 is connected drain-to-source across the motor/track terminals in a low-side configuration. When Q1 conducts, it presents a near-zero impedance path across the motor, absorbing the regenerative energy from the spinning motor.

Q1 is driven via TIM3_CH1 with INVERTED output polarity. This is the most important hardware design decision in the module and its rationale must be understood before any PCB layout:

| TIM3_CH1 CCR value | Output pin state | Gate driver output | Q1 state | Motor sees |
| --- | --- | --- | --- | --- |
| 0 (BRAKE_CCR_HARD) | Permanently HIGH | HIGH | Fully ON | Dead short |
| 2720 (BRAKE_CCR_SOFT) | PWM ~85% HIGH | PWM | Switching | ~8Ω effective |
| 3199 (BRAKE_CCR_OFF) | Permanently LOW | LOW | Fully OFF | Open circuit |
| MCU unpowered/reset | Floating → pull-up holds HIGH | HIGH | Fully ON | Dead short (safe) |

The inverted polarity means CCR=0 produces maximum output and CCR=ARR produces zero output, which is the inverse of the normal PWM convention. The firmware therefore maps higher pot values (CW, more braking) to lower CCR values. This is counterintuitive in code but the hardware consequence is critical: a floating or undriven gate defaults to the safe (braked) state via the 10kΩ pull-up on the gate driver input.

The 10kΩ pull-up resistor must be placed on the gate driver INPUT, not on Q1’s gate directly. The gate driver has a low-impedance push-pull output that fights any resistor placed after it. A resistor on the driver input is seen as a high impedance by the driver output and correctly biases the input to the logic-high state when the MCU output is floating.

## 11.2 Q2 — Keep-alive / Antibrake FET (P-channel, high-side)

Q2 is connected in a high-side configuration between the 12.4V rail and the track terminal. When Q2 conducts, it injects a fraction of the rail voltage onto the track. The effective injected voltage is controlled by the PWM duty cycle on TIM3_CH2.

| TIM3_CH2 CCR value | Effective voltage | Mode A use | Mode C use |
| --- | --- | --- | --- |
| 0 (KA_CCR_OFF) | 0V | FET off | Not used (ka always > 0) |
| ~258 (approx 1V) | ~1V | Light antibrake | Low keep-alive |
| ~400 (KA_CCR_MAX) | ~2V at 16V rail | Maximum anti-brake | Maximum keep-alive |
| 774 (KA_CCR_MAX) | ~3V | Maximum antibrake | Maximum keep-alive |

A P-channel FET is used for Q2 because it allows a simple high-side switch without a bootstrap circuit. The gate driver output drives the gate low to turn the FET on (P-channel convention). TIM3_CH2 uses normal (non-inverted) polarity: CCR=0 produces a LOW output, turning Q2 on; increasing CCR reduces duty, reducing the effective injected voltage.

## 11.3 Simultaneous Q1 and Q2 conduction (Mode C only)

In modes A and B, Q1 and Q2 are never on at the same time. In mode C they are: Q2 runs continuously while Q1 switches under wiper control. This creates an electrical conflict during hard braking that must be managed by the PCB layout.

When Q1 is fully on (dead short, CCR=0), the track terminals are held at approximately 0V. Q2 is simultaneously driving current from the 12.4V rail into this near-zero impedance. The energy dissipated in Q2 during this overlap is:

V_Q2 = V_RAIL − V_track ≈ 12.4V (worst case dead short)

I_Q2  = V_Q2 / (R_Q2_ds + R_track_load)

P_Q2  = V_Q2 × I_Q2  (for duration of Q1 on-time)

The keep-alive current is inherently small because the eCom MCU load is in the milliamp range. However, the instantaneous current during the moment Q1 turns on can be higher. The Schottky diode in series with Q2’s output limits this by blocking reverse current and clamping the voltage excursion on Q2’s drain when Q1 switches.

Recommended BOM addition for mode C boards: one SS34 Schottky diode (3A, 40V) in series with Q2’s source/drain output, oriented to pass current from Q2 toward the track terminal and block current flowing back toward Q2.

# 12. Updated Pin Map — STM32G041K6U6 (QFN-32)

## 12.1 Rationale for package change

The original design used the STM32G041J6M6 in an SOIC-8 package with only 6 usable GPIO pins. Adding the three-mode toggle switch (two pins), capture button (one ADC pin), and status LED (one pin) required four additional signals, making the SOIC-8 unsuitable.

The STM32G041K6U6 in QFN-32 was selected as the replacement. It uses the identical Cortex-M0+ core, the same 32K flash and 8K RAM, and runs the same firmware binary without modification. The QFN-32 package provides 20 usable GPIO pins, giving substantial headroom for future additions. The package is compact (5×5mm) and solderable with hot-air rework equipment.

## 12.2 Full pin assignment

| Pin | Signal | Type | Function |
| --- | --- | --- | --- |
| PA0 | Rail sense | ADC_IN0 analog | 12.4V rail ÷ voltage divider → ≤3.3V |
| PA1 | Wiper strip | GPIO input, pull-up | Active LOW — trigger wiper contact |
| PA2 | Toggle bit 0 | GPIO input, pull-up | LOW = mode B or invalid; decoded with PA4 |
| PA3 | Pot wiper | ADC_IN3 analog | 10k pot, 0–3.3V |
| PA4 | Toggle bit 1 | GPIO input, pull-up | LOW = mode C or invalid; decoded with PA2 |
| PA5 | Capture button | ADC_IN5 analog | 10k pull-up; button → GND via 1k. Unpressed ~4095, pressed ~372 |
| PA6 | Brake PWM | TIM3_CH1 AF1 | Q1 gate via GD1. INVERTED polarity. |
| PA7 | Keep-alive PWM | TIM3_CH2 AF1 | Q2 gate via GD2. Normal polarity. |
| PB6 | Status LED | GPIO output | Blinks 3× on successful keep-alive save to flash |

## 12.3 Toggle switch decoding

| PA2 state | PA4 state | Mode selected | Notes |
| --- | --- | --- | --- |
| HIGH (unpressed) | HIGH (unpressed) | A — Brushed + antibrake | Both pins floating high |
| LOW (pressed) | HIGH (unpressed) | B — Brushed, brake only | Bit 0 asserted |
| HIGH (unpressed) | LOW (pressed) | C — Brushless keep-alive | Bit 1 asserted |
| LOW (pressed) | LOW (pressed) | B (safe default) | Invalid combination — firmware defaults to mode B |

## 12.4 Capture button ADC circuit

The capture button uses a single ADC pin rather than a dedicated GPIO. A 10kΩ pull-up resistor connects PA5 to 3.3V. The capture button connects PA5 to GND via a 1kΩ series resistor. This produces two well-separated ADC voltage levels:

| Button state | V_PA5 | ADC count (12-bit) | Firmware interpretation |
| --- | --- | --- | --- |
| Unpressed | 3.3V | ~4095 | Not pressed |
| Pressed | 1k/(10k+1k) × 3.3V = 0.3V | ~372 | Pressed |
| Detection threshold | Midpoint ~1.8V | ~2200 | BTN_PRESSED_THRESHOLD constant |

Using ADC for the button rather than a GPIO has two advantages: the same ADC scan that reads the pot and rail also reads the button, with no additional interrupt or polling overhead; and the analogue threshold can be adjusted in firmware (BTN_PRESSED_THRESHOLD) to accommodate component tolerance without a hardware change.

# 13. Brake Ramp-In and Industry Terminology

## 13.1 Industry context

In slot car racing, three distinct braking techniques are in common use, and the terminology is sometimes used inconsistently by different manufacturers. The AART module implements all three, and the mode names have been updated to match industry convention:

| AART mode | Industry term | Description |
| --- | --- | --- |
| Mode B | Reduced braking | Variable resistance inserted across motor terminals during braking. No voltage applied. Most common implementation — some manufacturers incorrectly call this anti-brake. |
| Mode A (brake side) | Reduced braking | Same as mode B on the CCW half of the pot. Dead short at the centre detent. |
| Mode A (anti-brake side) | Positive anti-brake | Small forward voltage applied to motor when trigger released. Creates gentle forward torque opposing the braking force. Used on advanced controllers from Difalco, Third Eye Technology, and others. |
| Mode C | eCom keep-alive | Continuous low voltage maintains eCom MCU power regardless of throttle state. Specific to brushless motor installations. |

The AART module uses direct track power (WHITE rail) for all voltage injection. No stored energy (capacitors, batteries, supercapacitors) is used. This satisfies the direct track power requirement in current BSCRA and ISRA controller regulations, where energy storage devices that can continue driving the car after trigger release are typically prohibited. Verify against the specific class rules for the event.

## 13.2 Why positive anti-brake helps

When a high-downforce wing car releases the trigger at speed, a dead short across the motor creates an abrupt deceleration that can destabilise the car. The effect is described by racers as the car ‘tripping over’ or ‘snapping’ under braking.

Positive anti-brake applies a small forward torque (typically 0.2–2V) when the trigger is released. This:

- Reduces peak deceleration rate

- Keeps the rear tyres loaded during corner entry

- Makes the car feel ‘longer’ and smoother through fast sweepers

- Reduces deslotting risk under heavy braking

The effect is most noticeable with powerful motors on high-downforce cars and tracks with long flowing corners. On tight technical tracks with heavy braking zones, less anti-brake (or none) is usually preferred for corner precision.

## 13.3 Brake ramp-in

The brake ramp-in feature delays the onset of full braking after the trigger is released, creating an ‘active coast’ effect. Instead of applying the full pot-selected resistance immediately, the module ramps the brake CCR exponentially from zero toward the target value.

### 13.3.1 Exponential ramp shape

Each millisecond tick, the CCR moves a fraction of the remaining distance toward the target:

ccr += (target − ccr) × alpha / 256   (each 1ms tick)

alpha = 256  →  0ms  (immediate, one step)

alpha = 128  →  ~10ms to reach 95% of target

alpha = 26   →  ~50ms to reach 95% of target

alpha = 3    →  ~200ms to reach 95% of target

The exponential shape gives a fast initial brake onset (preventing the car running wide) followed by a gentler tail-off, which avoids the snap-stop that upsets high-downforce cars. This matches the feel of the ‘active coast’ circuits found in advanced commercial controllers.

### 13.3.2 Setting the ramp time

The ramp time is set using the capture button in modes A and B:

- Set the mode toggle to A or B.

- Hold the capture button. The brake is suspended.

- Rotate the pot: CCW = 0ms (immediate braking), CW = 200ms (slowest ramp).

- Release the button. The ramp time is saved to flash and the LED blinks 3×.

- The saved value persists across power cycles.

Starting point recommendations:

| Car type | Suggested ramp time | Rationale |
| --- | --- | --- |
| High-downforce wing car, powerful motor | 80–150ms | Prevents snap-stop on corner entry |
| Standard wing car | 30–80ms | Moderate onset softening |
| Pan car / inline | 0–30ms | Precise braking needed for tight corners |
| Brushless (mode C) | Not applicable | Ramp-in disabled in mode C — ESC needs prompt braking |

### 13.3.3 Interaction with positive anti-brake

In mode A, the ramp-in applies only to the brake (CCW) side of the pot. The positive anti-brake (CW side) takes effect immediately when the trigger releases and the pot is in the anti-brake zone. There is no ramp on the anti-brake injection — a gradual onset of forward torque would feel sluggish and is not beneficial.

## 13.4 Flash storage layout

The flash save record at page 31 (address 0x08007C00) now stores both the keep-alive CCR and the ramp time in a single 64-bit write:

Offset 0x00: uint32_t magic    = 0xAA270001

Offset 0x04: uint16_t ka_ccr   (bits 15:0  of upper word)

uint16_t ramp_ms  (bits 31:16 of upper word)

Full 64-bit record:

[63:48] = ramp_ms  (0–200)

[47:32] = ka_ccr   (0–400)

[31:0]  = magic    (0xAA270001)

If the magic word is not present (erased flash = all 0xFF), both values default to zero (immediate braking, keep-alive off). The capture procedure only needs to be performed once per motor/controller combination.

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | May 2026 | AART | Initial design document |
| 2.0 | May 2026 | AART | Added sections 8–9: libopencm3 firmware, Docker toolchain, G0-specific API notes, pre-commissioning checklist and scope verification sequence |
| 3.0 | May 2026 | AART | Added sections 10–12: three-mode design rationale, full FET behaviour reference with polarity explanation, keep-alive capture procedure, simultaneous Q1/Q2 conduction analysis, updated pin map for STM32G041K6U6 QFN-32 |
| 4.0 | May 2026 | AART | Updated section 9: scope verification tables split per mode, correct pot directions, expanded tuning table |
| 5.0 | Jun 2026 | AART | Major revision: sections 1–4 rewritten for Rev 4 architecture. Wiper strip removed; brake detection via BLACK terminal voltage sense with hysteresis (enter <500mV, exit >1500mV). Keep-alive suppressed when controller above KA setpoint. BLACK sense divider R10/R11 (22k/4k7) added. ADC scan extended to 4 channels. Rail voltage updated to 12–16V. Banana plug connections documented. |
| 6.0 | Jun 2026 | AART | Rev 5 firmware: exponential brake ramp-in added (0–200ms, saved to flash). Industry terminology adopted. Capture button dual-function. Flash record extended to 64-bit. BSCRA/ISRA legality note added. |
| 7.0 | Jun 2026 | AART | Comprehensive document audit: stale HAL-era constants replaced; title page updated; sections 4–6 corrected for V_BLACK brake detection; section 5 rewritten as Peripheral Configuration; section 7 open items refreshed; new section 7B Key Design Decisions added. |
| 8.0 | Jun 2026 | AART | Power and thermal sanity check applied. Component changes: D1 P6KE18A → P6KE18CA (bidirectional); D2/D3 SS34 → SS310 (100V); D4 SS34 → MBRS360; F1 25A → 500mA–1A WHITE rail only; R8 15k → 18k. New section 3.5 Power and Thermal Analysis added. |
| 9.0 | Jun 2026 | AART | Removed stale CubeMX Setup Checklist entirely. Duplicate stale bench test table removed. |
| 9.1 | Jun 2026 | AART | Section 4.3 corrected: HAL_ADC_ConvCpltCallback replaced with dma1_channel1_isr() (libopencm3 ISR); 'two-element buffer' updated to four channels. |