AART Slot Car Braking Module

Design Document

Target MCU: STM32G041J6M6 (SOIC-8)

Revision 1.0  |  AART — Adrian & Richard’s Technologies

# 1. Overview

This document describes the hardware design and firmware architecture for the AART slot car braking module. The module sits in-line between a conventional hand controller and the track, providing variable braking, freewheel, and antibrake functions under the control of a single centre-detent potentiometer.

## 1.1 Purpose

Some Group 12 motors with high Kv ratings stop too abruptly when a conventional dead-short brake is applied. The module addresses this by providing:

- Variable braking — from a hard short (full stop) to a soft ~8Ω effective resistance

- Freewheel — both FETs off, motor coasts freely

- Antibrake — a small forward voltage (0–2V) injected against the stopping force to smooth deceleration

## 1.2 System position

The module inserts between the existing hand controller output and the track feed:

| Stage | Signal | Notes |
| --- | --- | --- |
| Power supply | 12.4V DC rail (always live) | From club/track PSU |
| Hand controller | 0–12.4V variable DC | Throttle position controls voltage |
| This module | Pass-through + brake/antibrake | Activated by wiper strip contact |
| Track | Motor drive voltage | To car pickup rails |

The 12.4V rail is always present. The hand controller varies how much of it reaches the track. The module draws its MCU supply and antibrake injection voltage from the always-live rail.

# 2. Operating Modes

The module has four operating modes selected by the combination of wiper strip state and potentiometer position.

| Mode | Wiper strip | Pot position | Q1 brake FET | Q2 antibrake FET |
| --- | --- | --- | --- | --- |
| Passthrough | Open (throttle active) | Any | OFF | OFF |
| Hard brake | Closed (throttle released) | Full CCW | ON (hard short) | OFF |
| Variable brake | Closed | Below centre | PWM variable duty | OFF |
| Freewheel | Closed | Centre ±deadband | OFF | OFF |
| Antibrake | Closed | Above centre | OFF | PWM 0–16% duty |

## 2.1 Wiper strip

The hand controller has a wiper strip that makes contact at the lower end of the trigger travel (throttle fully released). This contact is the master enable for all braking functions. While the wiper is open (throttle active), both FETs are off and the module is fully transparent.

## 2.2 Brake region (pot below centre)

Q1 is a low-side N-channel FET connected across the motor/track terminals. PWM on Q1’s gate creates a variable effective resistance:

- CCR = 0 (FET fully on) → hard short across motor → maximum braking

- CCR = BRAKE_DUTY_SOFT → ~8Ω effective resistance → soft braking

The pot position maps linearly between these extremes.

## 2.3 Antibrake region (pot above centre)

Q2 injects a fraction of the 12.4V rail voltage into the motor to counteract back-EMF braking. Effective voltage is controlled by PWM duty cycle:

- Duty 0% → no injection

- Duty ~16% → approximately 2V effective at 12.4V supply

Q2 is a high-side switch. A P-channel FET (e.g. IRF9Z34N) is recommended for the prototype as it requires no bootstrap circuit. The gate drive polarity on TIM3_CH2 must be inverted to match.

## 2.4 Deadband (pot at centre)

A software deadband of ±150 ADC counts around the midpoint (ADC value 2048) creates a stable freewheel zone. Both FETs off, motor coasts. The deadband prevents mode chatter at the centre position.

# 3. Hardware Design

## 3.1 Component list

| Ref | Component | Value / Part | Notes |
| --- | --- | --- | --- |
| U1 | MCU | STM32G041J6M6 | SOIC-8, 3.3V, 64MHz |
| U2 | LDO regulator | e.g. LD1117-3.3 | 12V → 3.3V, 100mA |
| GD1 | Gate driver | UCC27524 or TC4427 | Brake FET channel |
| GD2 | Gate driver | UCC27524 or TC4427 | Antibrake FET channel |
| Q1 | Brake FET | IRLB3034 N-ch | 195A rated, 1.4mΩ Rds(on) |
| Q2 | Antibrake FET | IRF9Z34N P-ch | High-side switch, 18A |
| D1 | TVS diode | P6KE15A or similar | Rail transient protection |
| D2/D3 | Schottky diodes | SS34 or similar | Catch diodes across FETs |
| F1 | Fuse | 25A automotive | Rail protection |
| RV1 | Potentiometer | 10k linear, centre-detent | Control input |
| R1 | Pull-up resistor | 10kΩ | GD1 input pull-up (safe default) |
| R2/R3 | Rail divider | 15kΩ / 4.7kΩ | 12.4V → 2.96V for PA0 |
| C1–C4 | Decoupling caps | 100nF + 10μF per rail | Near each IC supply pin |

## 3.2 Pin assignments (STM32G041J6M6 SOIC-8)

| Pin | Signal | Direction | Function |
| --- | --- | --- | --- |
| PA0 | Rail sense | Input (ADC) | 12.4V rail divided to 2.96V via R2/R3 |
| PA1 | Wiper strip | Input (GPIO pull-up) | Active LOW — wiper contact pulls to GND |
| PA3 | Pot wiper | Input (ADC) | 10k pot centre tap, 0–3.3V |
| PA6 | TIM3_CH1 | Output (PWM) | Brake FET gate via GD1 |
| PA7 | TIM3_CH2 | Output (PWM) | Antibrake FET gate via GD2 |

Verify PA6 (TIM3_CH1 AF1) and PA7 (TIM3_CH2 AF1) are bonded and available in the J6M6 SOIC-8 variant before PCB layout. Use STM32CubeMX pin conflict checker.

## 3.3 Rail sense voltage divider

PA0 must not exceed 3.3V. The 12.4V rail is divided using:

- R2 = 15kΩ (top, to rail)

- R3 = 4.7kΩ (bottom, to GND)

Resulting voltage at PA0: 12.4 × 4.7 / (15 + 4.7) = 2.96V. Update ADC_RAIL_DIVIDER in brake_module.h to 0.2386 to match this divider ratio.

## 3.4 Safe default behaviour

On power loss or MCU fault, the brake FET (Q1) must default to ON (hard brake). This is achieved by:

- A 10kΩ pull-up resistor (R1) on the GD1 input. When the MCU output goes high-impedance (reset/power-off), R1 holds GD1 input HIGH → driver output HIGH → Q1 gate HIGH → FET on → hard brake.

- TIM3_CH1 is configured with INVERTED polarity (OCPolarity LOW) so CCR=0 produces a HIGH output during normal operation, consistent with the pull-up logic.

- BrakeModule_ForceSafeBrake() writes directly to CCR registers and is called from HardFault_Handler and NMI_Handler.

# 4. Firmware Architecture

## 4.1 File structure

| File | Description |
| --- | --- |
| Core/Inc/brake_module.h | Constants, pin map, types, public API |
| Core/Src/brake_module.c | Control logic, ADC, PWM management |
| Core/Src/peripheral_init.c | CubeMX peripheral configuration reference |
| Core/Src/main.c | Top-level integration |

## 4.2 Timing and scheduling

The module uses a polling architecture rather than RTOS. BrakeModule_Tick() is called from the main loop as fast as possible and uses HAL_GetTick() for internal pacing at 1ms intervals. ADC conversions run via DMA so the CPU never blocks waiting for a result.

## 4.3 ADC configuration

ADC1 is configured in scan mode with two channels in sequence:

- Rank 1 — PA0 (CH0): rail voltage sense

- Rank 2 — PA3 (CH3): pot wiper

DMA transfers results to a two-element buffer. A HAL_ADC_ConvCpltCallback sets a flag which the main tick reads on the next iteration. Sampling time is 39.5 cycles — adequate for the 10kΩ pot source impedance.

## 4.4 PWM configuration

TIM3 runs at 20kHz (above the audio band, good for logic-level FETs):

SYSCLK = 64 MHz, PSC = 0, ARR = 3199

Frequency = 64,000,000 / (0+1) / (3199+1) = 20,000 Hz

CH1 polarity: INVERTED (OCPolarity_LOW)

CCR = 0    -> output HIGH -> brake FET ON  (hard brake)

CCR = 3199 -> output LOW  -> brake FET OFF (no brake)

CH2 polarity: NORMAL (for P-channel Q2, set INVERTED)

CCR = 0   -> output LOW  -> antibrake FET OFF

CCR = 516 -> output HIGH -> ~2V effective (16% of 12.4V)

## 4.5 Key constants

| Constant | Default | Meaning |
| --- | --- | --- |
| ADC_CENTRE | 2048 | Midpoint of 12-bit ADC range |
| ADC_DEADBAND | 150 | ±3.7% of range — freewheel zone around centre |
| BRAKE_DUTY_HARD | 0 | CCR for full hard short (FET fully on) |
| BRAKE_DUTY_SOFT | 2720 | CCR for ~8Ω effective resistance at pot minimum |
| ANTI_DUTY_MAX | 516 | CCR for ~2V antibrake injection at 12.4V rail |
| WIPER_DEBOUNCE_MS | 10 | Wiper strip debounce time in milliseconds |
| ADC_RAIL_DIVIDER | 0.2386 | Voltage divider ratio R3/(R2+R3) for rail sense |
| RAIL_UNDERVOLTAGE_MV | 5000 | Below this rail voltage — force safe brake state |

## 4.6 Control flow

Each 1ms tick executes the following sequence:

- Update ADC readings from DMA buffer if conversion complete

- Check rail voltage — force safe brake if below RAIL_UNDERVOLTAGE_MV

- Read debounced wiper strip state (PA1)

- If wiper open: set MODE_PASSTHROUGH, both FETs off, return

- Read pot ADC value and compare against deadband thresholds

- Below deadband: compute brake duty, update TIM3_CH1, set antibrake to 0

- Above deadband: compute antibrake duty, update TIM3_CH2, set brake to full off

- Within deadband: both FETs off — MODE_FREEWHEEL

# 5. CubeMX Setup Checklist

Configure the following in STM32CubeMX before generating code. DMA must be initialised before ADC in the generated code — check the call order in main.c.

| Peripheral | Setting | Value |
| --- | --- | --- |
| RCC | PLL source | HSI16 |
| RCC | SYSCLK | 64 MHz (PLLN=8, PLLP=2, PLLR=2) |
| TIM3 | CH1 pin | PA6 — verify on J6M6 SOIC-8 |
| TIM3 | CH2 pin | PA7 — verify on J6M6 SOIC-8 |
| TIM3 | PSC | 0 |
| TIM3 | ARR | 3199 |
| TIM3 | CH1 polarity | INVERTED (LOW active) — critical for safe default |
| TIM3 | CH2 polarity | Normal HIGH (or inverted if P-channel Q2) |
| ADC1 | Scan mode | Enabled |
| ADC1 | Channels | IN0 (PA0) rank 1, IN3 (PA3) rank 2 |
| ADC1 | Trigger | Software |
| ADC1 | DMA | DMA1 CH1, Circular, Word width |
| PA1 | Mode | Input, Pull-up |

# 6. Bench Testing

## 6.1 Oscilloscope check points

| Test condition | PA6 (brake) | PA7 (antibrake) | Expected |
| --- | --- | --- | --- |
| Power on, wiper open | HIGH | LOW | Hard brake default active |
| Wiper closed, pot full CCW | HIGH | LOW | Hard short |
| Wiper closed, pot below centre | 20kHz PWM | LOW | Variable duty 0–85% |
| Wiper closed, pot at centre | LOW | LOW | Both FETs off |
| Wiper closed, pot above centre | LOW | 20kHz PWM | 0–16% duty |
| Wiper open (any pot) | LOW | LOW | Module transparent |

## 6.2 Power budget

At 20A motor current with IRLB3034 (Rds(on) = 1.4mΩ typical):

P_FET = I² × Rds(on) = 400 × 0.0014 = 0.56W per FET

Well within D2PAK/SOT-223 ratings. A small heatsink or copper pour under the thermal pad is good practice for sustained use.

# 7. Open Items

| Item | Issue | Recommended action |
| --- | --- | --- |
| Q2 topology | UCC27524 is a low-side driver; Q2 is high-side | Use P-channel FET (IRF9Z34N) and invert TIM3_CH2 polarity |
| PA6/PA7 availability | J6M6 SOIC-8 pin bonding unverified | Confirm with CubeMX pin conflict checker before PCB |
| Pull-up vs gate driver | 10k pull-up fights UCC27524 output during normal operation | Place pull-up before gate driver input, not on gate directly |
| ADC_RAIL_DIVIDER | Default value assumes 15k/4.7k divider | Measure actual resistors and update constant accordingly |

# 8. Firmware Build — libopencm3 and Docker Toolchain

## 8.1 Library choice

The firmware was written against libopencm3 rather than the ST HAL. This eliminates the CubeMX dependency, keeps the entire toolchain open source, and produces a significantly smaller binary. The compiled debug image is 4,616 bytes of text — approximately 14% of the available 32K flash.

libopencm3 is included as a git submodule and pre-built inside the Docker image for the STM32G0 family only, keeping the image lean. No ST account, no proprietary downloads, no generated code blobs are required to build the project from scratch.

## 8.2 G0-specific differences from other STM32 families

Several libopencm3 API differences apply specifically to the STM32G0 and are worth documenting here for anyone porting from F-series examples:

- DMAMUX — The G0 DMA controller routes requests through a DMAMUX peripheral. Unlike F0/F1 where the DMA channel-to-peripheral mapping is fixed in hardware, the G0 requires an explicit write to DMAMUX1_C0CR to connect ADC1 to DMA1 Channel 1. In the firmware this is done with a direct MMIO32 write to address 0x40020800 (DMAMUX1 base, channel 0 register).

- ADC scan mode — adc_enable_scan_mode() does not exist for G0. Scan is implicit when the sequence length is greater than one. Set the sequence with adc_set_regular_sequence() and the hardware scans automatically.

- ADC resolution symbol — ADC_RESOLUTION_12BIT (F-series) becomes ADC_CFGR1_RES_12_BIT on G0.

- ADC sampling time symbol — ADC_SMPR_SMP_39DOT5CYC becomes ADC_SMPR_SMPx_039DOT5CYC on G0.

- ADC DMA continuous — adc_set_dma_continue() becomes adc_enable_dma_circular_mode() on G0.

- Timer reset — timer_reset() is not available for G0 timers. Use rcc_periph_reset_pulse(RST_TIM3) instead.

- Flash wait states — The G0 rcc_clock_scale struct takes a plain integer (2) for flash_waitstates, not the FLASH_ACR_LATENCY_2WS macro.

- Linker script — libopencm3 dropped all per-device .ld files in November 2020 in favour of a script generator. The project provides its own stm32g041j6.ld containing the MEMORY block (32K flash at 0x08000000, 8K RAM at 0x20000000) and an INCLUDE cortex-m-generic.ld directive.

## 8.3 Docker toolchain

The build environment is fully containerised and reproducible. The Docker image is multi-stage:

| Stage | Contents | Build command |
| --- | --- | --- |
| base | Debian Bookworm slim + apt packages including OpenOCD 0.12.0 | (base of all stages) |
| toolchain | ARM GCC 13.3.rel1 (arm64 native) + libopencm3 pre-built for G0 | docker build --target toolchain -t aart-brake:toolchain . |
| cubeprog | toolchain + STM32CubeProgrammer (optional, needs ST installer) | docker build --target cubeprog -t aart-brake:full . |

OpenOCD is installed from the Debian Bookworm apt repository rather than built from source. The upstream GitHub release for v0.12.0 does not attach a source tarball (only auto-generated archives without a configure script), making source builds unreliable. The Debian package provides the same version without this problem.

## 8.4 Build workflow

All build, flash, and debug operations are available as simple commands:

# Compile (debug)

docker compose run --rm brake build

# Compile (release, optimised)

docker compose run --rm brake build release

# Flash via ST-Link (from host, recommended on macOS)

st-flash write build/aart_brake.bin 0x08000000

# Or via OpenOCD on the host

openocd -f interface/stlink.cfg -f target/stm32g0x.cfg \

-c "program build/aart_brake.elf verify reset exit"

On macOS, Docker Desktop does not reliably pass USB devices to containers. Build inside Docker, then flash from the host using st-flash (brew install stlink) or host OpenOCD (brew install openocd). The build/ directory is bind-mounted so the .elf/.bin/.hex files are available on the host immediately after a container build.

# 9. Pre-Commissioning Checklist

## 9.1 Before powering up

Work through these checks before applying track supply voltage for the first time:

- Confirm PA6 and PA7 are bonded and support TIM3_CH1/CH2 AF1 in the STM32G041J6M6 SOIC-8 variant — verify against the device datasheet, not just the family reference manual.

- Verify the voltage divider resistors for the rail sense input (PA0). Measure the actual fitted resistor values and update ADC_RAIL_DIVIDER in brake_module.h before building. The default assumes 15kΩ and 4.7kΩ; a wrong value will misread the rail voltage and may trigger spurious safe-brake events.

- Confirm the 10kΩ pull-up on the gate driver input (GD1) is placed before the gate driver input pin, not on the FET gate directly. The gate driver has a low-impedance output that will fight a pull-up placed after it.

- Resolve the Q2 antibrake FET topology before PCB layout. The UCC27524 is a low-side gate driver and cannot bootstrap above its supply rail. For the prototype use a P-channel FET (e.g. IRF9Z34N) and invert TIM3_CH2 polarity to OCPOLARITY_LOW to match.

- Check catch/freewheeling Schottky diodes are in place across both FET drain-source junctions. At 20A with an inductive load, omitting these will destroy the FETs on the first switching edge.

## 9.2 Scope verification sequence

With the MCU powered but no track supply connected, probe PA6 and PA7 and work through each state:

| Step | Action | PA6 (brake) | PA7 (antibrake) | Pass condition |
| --- | --- | --- | --- | --- |
| 1 | Power on, wiper open (throttle active) | LOW | LOW | Module transparent — no FET drive |
| 2 | Close wiper, pot full CCW | HIGH (DC) | LOW | Hard brake — FET fully on |
| 3 | Close wiper, pot halfway below centre | 20kHz PWM ~40% | LOW | Variable brake — measure duty with scope |
| 4 | Close wiper, pot at centre detent | LOW | LOW | Freewheel — both FETs off |
| 5 | Close wiper, pot halfway above centre | LOW | 20kHz PWM ~8% | Antibrake — low duty injection |
| 6 | Close wiper, pot full CW | LOW | 20kHz PWM ~16% | Max antibrake (~2V effective) |
| 7 | Remove MCU power (simulate loss) | HIGH (via pull-up) | LOW | Safe default — brake on without MCU |

Step 7 is the most important safety verification. With the MCU unpowered, the 10kΩ pull-up on the GD1 input must hold the gate driver input high, causing GD1 to drive the brake FET gate high and short the motor. If PA6 does not go high when the MCU loses power, the safe-default circuit is not working and the module must not be used on the track.

## 9.3 On-track tuning

Once scope verification passes, the following constants in brake_module.h can be tuned on the bench with a motor under load:

| Constant | Effect | Tune by |
| --- | --- | --- |
| ADC_DEADBAND | Width of freewheel zone around pot centre | Increase if mode flickers at centre; decrease for finer control |
| BRAKE_CCR_SOFT | Minimum braking strength (pot at bottom of brake range) | Adjust until motor stops within acceptable distance at minimum brake |
| ANTI_CCR_MAX | Maximum antibrake injection voltage (~2V default) | Reduce if motor still over-brakes; increase if effect is too subtle |
| WIPER_DEBOUNCE_MS | Wiper strip contact debounce time | Increase to 20ms if mode flickers on trigger release |

# Revision History

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | May 2026 | AART | Initial design document |
| 2.0 | May 2026 | AART | Added sections 8–9: libopencm3 firmware, Docker toolchain, G0-specific API notes, pre-commissioning checklist and scope verification sequence |